STANDARD MICROSYSTEMS

8003/8013 FAMILY LAN ADAPTERS

JUMPER AND CONFIGURATION SETTINGS

November 22, 1991

Each Standard Microsystems (formerly Western Digital) LAN adapter

provides several parameters that may need to be configured prior

to using the adapter in a network.  Some adapters are configured

by setting appropriate jumpers on the board, others are configured

via a software setup program.  The following tables summarize the

configuration options for each adapter.

For more detailed information on any adapter, or for information on

jumpers for an adapter that is not listed here, refer to the User

Installation Guide that comes with the adapter.

Notes regarding XT/AT bus adapters:

o Each "PLUS"-family LAN adapter board contains a RAM buffer

of 8, 16, or 32 Kbytes that occupies PC memory space.  The

base address for this RAM memory is set by the software

driver, or the adapter configuration utility you use,

not by jumpers on the board.

o Other add-on boards may introduce conflicts with this memory

space.  Also, system conflicts can occur with the I/O base

address and interrupt request level, which are controlled by

jumpers or are software configurable as described below.

For a Micro Channel bus board, the RAM base address, I/O base

address, boot ROM base address, and interrupt request level are

set by the system configuration procedure when the board is

installed.  Every software driver obtains this information

automatically and adjusts its operation accordingly.

"*" DENOTES THE SETTINGS OF JUMPERS INSTALLED AT THE FACTORY.

== EtherCard PLUS ELITE (SMC 8003EP and WD8003EP) ==

JUMPER

BLOCK    NAME    POSITION                     FUNCTION

------   ------  -----------  ------------------------------------------

W1      CONFIG  SOFT         If jumper is installed when power is turned

on, the I/O Base Address, IRQ, and RAM Base

Address are set to the value last stored

with the EZSETUP program.  Initially the I/O

Base Address will be 240 with RAM disabled.

280,3,D0000* If jumper is installed when power is turned

on:

-I/O Base Address is set to 280.

-IRQ is set to 3.

-RAM Base Address is set to D0000.

300,5,CA000  If jumper is installed when power is turned

on:

-I/O Base Address is set to 300.

-IRQ is set to 5.

-RAM Base Address is set to CA000.

W2      ROM     NONE/SOFT*   If jumper is installed when power is turned

ADDR                 on:

-the ROM Base Address is set to the value

last stored, or is disabled with the EZSETUP

program.

D8000        If jumper is installed when power is turned

on,

-ROM Base Address is set to D8000.

W3      AUI-                 Ethernet type:  use AUI position for thick

BNC                  Ethernet (external transceiver); use BNC (*)

position for thin Ethernet (BNC connector).

== EtherCard PLUS ELITE16 (SMC 8013EPC and WD8013EP) ==

JUMPER

BLOCK    NAME    POSITION                     FUNCTION

------   ------  -----------  ------------------------------------------

W1      CONFIG  SOFT         If jumper is installed when power is turned

on, the I/O Base Address, IRQ, and RAM Base

Address are set to the value last stored

with the EZSETUP program.  Initially the I/O

Base Address will be 240 with RAM disabled.

280,3,D0000* If jumper is installed when power is turned

on:

-I/O Base Address is set to 280.

-IRQ is set to 3.

-RAM Base Address is set to D0000.

300,10,CC000  If jumper is installed when power is turned

on:

-I/O Base Address is set to 300.

-IRQ is set to 10.

-RAM Base Address is set to CC000.

W2      ROM     SOFT*        If jumper is installed when power is turned

ADDR                 on:

-the ROM Base Address is set to the value

last stored, or is disabled with the EZSETUP

program.

D8000        If jumper is installed when power is turned

on,

-ROM Base Address is set to D8000.

W3      AUI-                 Ethernet type:  use AUI position for thick

BNC                  Ethernet (external transceiver); use BNC (*)

position for thin Ethernet (BNC connector).

NOTE! The SMC 8013EPC does not contain

a W3 jumper, cable type is configured

through EZSETUP.

== EtherCard PLUS ELITE16C (SMC 8013EWC and WD8013EW) ==

JUMPER

BLOCK    NAME    POSITION                     FUNCTION

------   ------  -----------  ------------------------------------------

W1      CONFIG  SOFT         If jumper is installed when power is turned

on, the I/O Base Address, IRQ, and RAM Base

Address are set to the value last stored

with the EZSETUP program.  Initially the I/O

Base Address will be 240 with RAM disabled.

280,3,D0000* If jumper is installed when power is turned

on:

-I/O Base Address is set to 280.

-IRQ is set to 3.

-RAM Base Address is set to D0000.

300,10,CC000  If jumper is installed when power is turned

on:

-I/O Base Address is set to 300.

-IRQ is set to 10.

-RAM Base Address is set to CC000.

W2      ROM     SOFT*        If jumper is installed when power is turned

ADDR                 on:

-the ROM Base Address is set to the value

last stored, or is disabled with the EZSETUP

program.

D8000        If jumper is installed when power is turned

on,

-ROM Base Address is set to D8000.

W3      AUI-                 Ethernet type:  use AUI position for thick

BNC                  Ethernet (external transceiver); use BNC (*)

position for thin Ethernet (BNC connector).

NOTE! The SMC 8013EWC does not contain

a W3 jumper, cable type is configured

through EZSETUP.

== EtherCard PLUS ELITE16T (SMC 8013WC and WD8013W) ==

JUMPER

BLOCK    NAME    POSITION                     FUNCTION

------   ------  -----------  ------------------------------------------

W1      CONFIG  SOFT         If jumper is installed when power is turned

on, the I/O Base Address, IRQ, and RAM Base

Address are set to the value last stored

with the EZSETUP program.  Initially the I/O

Base Address will be 240 with RAM disabled.

280,3,D0000* If jumper is installed when power is turned

on:

-I/O Base Address is set to 280.

-IRQ is set to 3.

-RAM Base Address is set to D0000.

300,10,CC000  If jumper is installed when power is turned

on:

-I/O Base Address is set to 300.

-IRQ is set to 10.

-RAM Base Address is set to CC000.

W2      ROM     SOFT*        If jumper is installed when power is turned

ADDR                 on:

-the ROM Base Address is set to the value

last stored, or is disabled with the EZSETUP

program.

D8000        If jumper is installed when power is turned

on,

-ROM Base Address is set to D8000.

== EtherCard PLUS with Boot ROM Socket (WD8003EB) ==

JUMPER

BLOCK    NAME                  FUNCTION

------   ----    ---------------------------------------------

W1      INIT    If jumper is installed when power is turned on,

280     I/O Base Address is temporarily set to 280.

Setup Program must then be run to store new I/O

Base Address, and jumper must be removed (*).

W3      AUI-    Ethernet type:  use AUI position for thick

BNC     Ethernet (external transceiver); use BNC (*)

position for thin Ethernet (BNC connector).

W9      ROM     Size of optional ROM installed (2 jumpers).

Size                  LEFT JUMPER     RIGHT JUMPER

POSITION         POSITION

16 Kbytes(*)      Top              Top

32 Kbytes       Bottom             Top

64 Kbytes       Bottom           Bottom

Jumpers are ignored if no ROM is installed

and ROM is disabled through EZSETUP.

== EtherCard PLUS16 16-Bit Ethernet Adapter (WD8013EBT) ==

JUMPER

BLOCK   NAME                    FUNCTION

------  --------  --------------------------------------------

W0      Wait      Reduces the required wait states for an 8-

States    bit memory access from four to two.

8-Bit

Access

W0      Wait      Reduces the required wait states for a 16-

State     bit memory access from one to zero.

16-Bit

Access    Install both the 8-and 16-bit jumpers to

enhance performance on systems with I/O

bus speeds of 8MHz or less.  Factory setting

is both jumpers removed, which ensures

correct operation in faster bus systems.

The jumpers are ignored in XT bus systems.

W1      I/O Base  Selects the hex I/O Base Address.  Options

Address   are  200,  220,  240,  260,  280(*),  2A0,

2C0,  300,  340, and 380

W2      IRQ       Selects the interrupt request (IRQ) level

Level     to be used by the adapter.  IRQ10, 11 and

15 are not available in 8-bit bus systems

such as the IBM PC and PC XT.  IRQ2/9

position selects IRQ2 for PC and XT type

systems (8-bit bus), and IRQ9 for AT type

systems (16-bit bus).  Only one jumper

may be installed at a time.

IRQ options are:  2, 3(*), 4, 5, 7,

9, 10, 11, 15

W3      AUI-      Ethernet type:  Use AUI position for thick

BNC       Ethernet (external transceiver): Use BNC(*)

position for thin Ethernet (BNC connector).

W6      ROM       Selects ROM memory space, if a ROM is

Memory    installed.  Options:  NONE(*), 16K, 32K,

Space     and 64K

W9      ROM       Size of optional ROM installed (2 jumpers).

Size      Jumpers are ignored when W6 is set to NONE.

ROM SIZE   TOP JUMPER   BOTTOM JUMPER

POSITION      POSITION

16 Kbytes(*)    Left          Left

32 Kbytes       Left          Right

64 Kbytes       Right         Right

W15       ROM       Establishes the base address for the ROM

Base      device (Up to 3 jumpers).  Jumpers are

Address   ignored when W6 is set to NONE.

SELECTED       JUMPERS INSTALLED AT:

ADDRESS    C4000        C8000       D0000

C0000

C4000        X

C8000                     X

D0000                                 X

CC000        X            X

D4000        X                        X

D8000(*)                  X           X

DC000        X            X           X

NOTE! X - Represents an installed jumper.

== EtherCard PLUS (WD8003E) ==

And

== EtherCard PLUS With Boot ROM Socket (WD8003EBT) ==

JUMPER  CLOSED           FUNCTION SELECTED

BLOCK   PIN #'S   NAME   WHEN JUMPER IS INSTALLED

------  --------  ----   ----------------------------

W1       1-2      Zero   Select zero wait states for optimized

Wait   operation in a 6MHz AT.  (Factory

setting is jumper removed, which

ensures correct operation in faster

AT bus systems.)  This jumper is

ignored in XT bus systems.

W1                I/O Base  Selects the I/O Base Address

Address

3-4, 5-6, 7-8, 9-10  = 200

5-6, 7-8, 9-10       = 220

3-4, 7-8, 9-10       = 240

7-8, 9-10            = 260

3-4*,5-6*,9-10*      = 280 -- the factory setting

5-6, 9-10            = 2A0

3-4, 9-10            = 2C0

9-10                 = 2E0

3-4, 5-6, 7-8        = 300

5-6, 7-8             = 320

3-4, 7-8             = 340

7-8                  = 360

3-4, 5-6             = 380

5-6                  = 3A0

3-4                  = 3C0

NONE                 = 3E0

W2       1-2      IRQ7   Interrupt request level.

3-4      IRQ6   Only one jumper may be installed at a

5-6      IRQ5   time.

7-8      IRQ4

9-10*    IRQ3   (Factory setting is IRQ3)

11-12    IRQ2

W3       1-2*     AUI-   Ethernet type: pins all jumpered to

3-4*     BNC    use J1 BNC connector for thin Ethernet

5-6*            cable (* - the factory setting).

7-8*            Jumpers all removed to use J2 AUI

9-10*           connector (thick Ethernet cable).

11-12*

W4       1-2   Ethernet  Jumper removed for thin Ethernet and

Version   IEEE 802.3/Version 2 thick Ethernet

(the factory setting);  jumper in-

stalled for thick Ethernet Version 1.

W5       1-2*   Segment  Selects thin Ethernet segment length:

Length   jumpered for IEEE 802.3 standard 185m

length (*-the factory setting); jumper

removed for 300m extended length.

== Additional Jumpers On ==

== EtherCard PLUS With ROM Socket Only (WD8003EBT) ==

JUMPER  CLOSED           FUNCTION SELECTED WHEN

BLOCK   PIN #'S   NAME   INDICATED JUMPERS ARE INSTALLED

------  -------   ----    ------------------------------------

W6      NONE      ROM    C0000 (16K, 32K, 64K ROMs)

1-2       Base   C4000 (16K ROMs)

3-4      Address C8000 (16K, 32K ROMs)

1-2, 3-4         CC000 (16K ROMs)

5-6              D0000 (16K, 32K, 64K ROMs)

1-2, 5-6         D4000 (16K ROMs)

3-4*, 5-6*       D8000 (16K, 32K ROMs)--factory setting

1-2, 3-4, 5-6    DC000 (16K ROMs)

(NOTE:  Do not use addresses C0000

or C4000 for Boot ROMs)

W6      7-8*,9-10* ROM   ROM Disabled -- the factory setting

W9      3-4*       Size     "

W10     3-4*           "

W6      NONE       ROM   16 Kbytes ROM

W9      3-4        Size     "

W10     3-4            "

W6      7-8        ROM   32 Kbytes ROM

W9      3-4        Size     "

W10     1-2            "

W6      9-10       ROM   64 Kbytes ROM

W9      1-2        Size     "

W10     1-2            "

W7      3-4*       RAM   8 Kbytes RAM -- the factory setting

W8      3-4*       Buffer   "

W11     2-3*       Size     "

W7      1-2        RAM   32 Kbytes RAM

W8      1-2        Buffer     "

W11     1-2        Size       "      (NOTE:  valid 32 Kbyte RAM

base addresses are C0000, C8000,

D0000 and D8000.)

== EtherCard PLUS TP For Unshielded Twisted Pair Wiring,

LattisNet Compatible (WD8003WT) ==

JUMPER  CLOSED           FUNCTION SELECTED

BLOCK   PIN #'S   NAME   WHEN JUMPERS ARE INSTALLED

------  --------  ----   -------------------------------------

W1       1-2      Zero   Select zero wait states for optimized

Wait   operation in a 6MHz AT.  (Factory

State  setting is jumper removed, which

ensures correct operation in faster

AT bus systems.)  This jumper is

ignored in XT bus systems.

W1             I/O Base  Selects the I/O Base Address

Address   (Up to 3 jumpers)

3-4, 5-6, 7-8, 9-10  = 200

5-6, 7-8, 9-10       = 220

3-4, 7-8, 9-10       = 240

7-8, 9-10            = 260

3-4*,5-6*,9-10*      = 280 -- the factory setting

5-6, 9-10            = 2A0

3-4, 9-10            = 2C0

9-10                 = 2E0

3-4, 5-6, 7-8        = 300

5-6, 7-8             = 320

3-4, 7-8             = 340

7-8                  = 360

3-4, 5-6             = 380

5-6                  = 3A0

3-4                  = 3C0

NONE                 = 3E0

W2       1-2      IRQ7   Selects  the Interrupt request level.

3-4      IRQ6   Only one jumper may be installed at a

5-6      IRQ5   time.

7-8      IRQ4

9-10*    IRQ3   -- the factory setting

11-12    IRQ2

W3       1-2*     RJ-45  Ethernet type: pins all jumpered to

3-4*     -AUI   use RJ-45 twisted pair wiring connec-

5-6*            tor (*-the factory setting).  Jumpers

7-8*            all removed to use AUI connector.

9-10*

11-12*

W6             ROM Base  Selects the ROM base address

Address   (Up to 3 jumpers)

NONE             C0000 (16K, 32K, 64K ROMs)

1-2              C4000 (16K ROMs)

3-4              C8000 (16K, 32K ROMs)

1-2, 3-4         CC000 (16K ROMs)

5-6              D0000 (16K, 32K, 64K ROMs)

1-2, 5-6         D4000 (16K ROMs)

3-4*, 5-6*       D8000 (16K, 32K ROMs)--factory setting

1-2, 3-4, 5-6    DC000 (16K ROMs)

(NOTE:  Do not use addresses C0000

or C4000 for Boot ROMs)

W6      7-8*,9-10* ROM   ROM Disabled -- the factory setting

W9      2-3*       Size     "

W10     2-3*           "

W6      NONE       ROM   16 Kbytes ROM

W9      2-3        Size     "

W10     2-3            "

W6      7-8        ROM   32 Kbytes ROM

W9      2-3        Size     "

W10     1-2            "

W6      9-10       ROM   64 Kbytes ROM

W9      1-2        Size     "

W10     1-2            "

W7      2-3*      RAM     8 Kbytes RAM -- the factory setting

W8      2-3*      Buffer    "

W11     2-3*      Size "

W7      1-2       RAM     32 Kbytes RAM

W8      1-2       Buffer    "

W11     1-2       Size      "       (NOTE:  valid 32 Kbyte RAM

base addresses are C0000, C8000, D0000

and D8000.)

== EtherCard PLUS10T for Unshielded Twisted Pair Wiring - 10BaseT

Draft Standard Compatible ( WD8003W ) ==

JUMPER

BLOCK   NAME                  FUNCTION

------  ----    ---------------------------------------------

W1      INIT    If jumper is installed when power is turned on,

280     I/O Base Address is temporarily set to 280.

Setup Program must then be run to store new I/O

Base Address, and jumper must be removed (*).

W9      ROM     Size of optional ROM installed (2 jumpers).

Size                  LEFT JUMPER     RIGHT JUMPER

POSITION         POSITION

16 Kbytes(*)      Top              Top

32 Kbytes       Bottom             Top

64 Kbytes       Bottom           Bottom

Jumpers are ignored if no ROM is installed.

W20       Auto  In the ON position (*), the adapter automati-

Polarity  cally corrects for incorrect polarity on the

Correction  receive twisted pair.  If W20 is installed in

the OFF position, the receive twisted pair

polarity must be correct on the RJ-45 port

according to the 10BaseT standard.

W21       Link  The 10BaseT link integrity test between the

Integrity  adapter and a 10BaseT concentrator is per-

formed with the jumper removed (*).  The

link integrity test is not performed if the

jumper is installed.

== EtherCard PLUS10T for Unshielded Twisted Pair Wiring - 10BaseT

Draft Standard Compatible ( SMC 8003WC ) ==

JUMPER

BLOCK    NAME    POSITION                     FUNCTION

------   ------  -----------  ------------------------------------------

W1      CONFIG  SOFT         If jumper is installed when power is turned

on, the I/O Base Address, IRQ, and RAM Base

Address are set to the value last stored

with the EZSETUP program.  Initially the I/O

Base Address will be 240 with RAM disabled.

280,3,D0000* If jumper is installed when power is turned

on:

-I/O Base Address is set to 280.

-IRQ is set to 3.

-RAM Base Address is set to D0000.

300,5,CA000  If jumper is installed when power is turned

on:

-I/O Base Address is set to 300.

-IRQ is set to 5.

-RAM Base Address is set to CA000.

W2      ROM     SOFT*        If jumper is installed when power is turned

ADDR                 on:

-the ROM Base Address is set to the value

last stored, or is disabled with the EZSETUP

program.

D8000        If jumper is installed when power is turned

on,

-ROM Base Address is set to D8000.

== EtherCard PLUS/A For Micro Channel (WD8003ET/A) ==

JUMPER  CLOSED           FUNCTION SELECTED WHEN

BLOCK   PIN #'S  NAME    INDICATED JUMPERS ARE INSTALLED

------  -------- ----    ------------------------------------

W3      1-2*      AUI-   Ethernet type: pins all jumpered to

3-4*      BNC    use J1 BNC connector for thin Ethernet

5-6*              cable (*-the factory setting).

7-8*             Jumpers all removed to use J2 AUI

9-10*             connector (thick Ethernet cable).

11-12*

W9      2-3*      ROM    16 Kbytes ROM -- the factory setting

W10     NONE*     Size      "

W9      1-2       ROM    32 Kbytes ROM

W10     1-2       Size      "

W9      2-3       ROM    64 Kbytes ROM

W10     1-2       Size      "

== EtherCard PLUS/A for Micro Channel (WD8003E/A) ==

JUMPER

BLOCK   NAME                  FUNCTION

------  ----    ---------------------------------------------

W3      AUI-    Ethernet type (2 jumpers):  Use AUI position

BNC     for both jumpers for thick Ethernet (external

transceiver);  Use BNC (*) position for both

jumpers for thin Ethernet (BNC connector).

W9,W10  ROM     Size of optional ROM installed (2 jumpers).

Size

W10           W9

16 Kbytes (*)    Top           Top

32 Kbytes       Bottom         Top

64 Kbytes       Bottom       Bottom

Jumpers are ignored if no ROM is installed

and ROM is disabled via the configuration diskette.

== EtherCard PLUS10T/A for Micro Channel (WD8003W/A) ==

JUMPER

BLOCK    NAME                  FUNCTION

------   ----   ---------------------------------------------

W9,W10   ROM    Size of optional ROM installed (2 jumpers).

Size

W10           W9

16 Kbytes (*)    Top           Top

32 Kbytes       Bottom         Top

64 Kbytes       Bottom        Bottom

Jumpers are ignored if no ROM is installed

and ROM is disabled via the configuration diskette.

W20       Auto  In the ON position (*), the adapter automati-

Polarity  cally corrects for incorrect polarity on the

Correction  receive twisted pair.  If W20 is installed in

the OFF position, the receive twisted pair

polarity must be correct on the RJ-45 port

according to the 10BaseT standard.

W21       Link  The 10BaseT link integrity test between the

Integrity  adapter and a 10BaseT concentrator is per-

formed with the jumper removed (*).  The

link integrity test is not performed if the

jumper is installed.

== StarCard PLUS/A For Micro Channel (WD8003ST/A) ==

JUMPER  CLOSED           FUNCTION SELECTED WHEN

BLOCK   PIN #'S   NAME   INDICATED JUMPERS ARE INSTALLED

------  --------  ----   ------------------------------------

W9      2-3*      ROM    16 Kbytes ROM--the factory setting

W10     NONE*     Size      "

W9      2-3       ROM    32 Kbytes ROM

W10     1-2       Size        "

W9      1-2       ROM    64 Kbytes ROM

W10     1-2       Size      "

== StarCard PLUS (WD8003S) ==

And

== StarLink PLUS (WD8003SH) ==

JUMPER  CLOSED           FUNCTION SELECTED

BLOCK   PIN #'S   NAME   WHEN JUMPER IS INSTALLED

------  --------  ----   ----------------------------

W2       1-2      Zero   Select zero wait states for optimized

Wait   operation in a 6MHz AT.  (Factory

setting is jumper removed, which

ensures correct operation in faster

AT bus systems.)  This jumper is

ignored in XT bus systems.

W2             I/O Base  Selects the I/O Base Address

Address   (Up to 3 jumpers)

3-4*,5-6*,9-10* 280 -- the factory setting

5-6, 9-10       2A0

3-4, 9-10       2C0

9-10            2E0

3-4, 5-6, 7-8   300

5-6, 7-8        320

3-4, 7-8        340

W1       1-2      IRQ7   Interrupt request level.

3-4      IRQ6   Only one jumper may be installed at a

5-6      IRQ5   time.

7-8      IRQ4

9-10*    IRQ3   -- the factory setting

11-12    IRQ2

W3       1-2*            StarCard PLUS (WD8003S):

Jumper must be installed

1-2             StarLink PLUS (WD8003SH):

Jumper must be removed

(for compatibility with 802.3 1BASE5

StarLAN standard.)

== Additional Jumpers On ==

== StarLink PLUS Only (WD8003SH) ==

JUMPER  CLOSED                      FUNCTION SELECTED WHEN

BLOCK   PIN #'S              NAME   INDICATED JUMPERS ARE INSTALLED

------  --------          --------- -----------------------------------

W4      11-12                ROM    Enabled when jumper installed;

Enable  disabled (the factory setting) when

jumper removed

W4                        ROM Base  Selects the ROM Base Address

Address  (Up to 3 jumpers)

3-4*,7-8*,9-10*     C8000   (16K, 32K ROMs)--factory setting

7-8, 9-10           CC000   (16K ROMs)

3-4, 5-6,9-10       D0000   (16K, 32K, 64K ROMs)

5-6, 9-10           D4000   (16K ROMs)

3-4, 9-10           D8000   (16K, 32K ROMs)

W5      1-3*, 2-4*          ROM     16 Kbytes ROM -- the factory setting

W6      1-3*, 2-4*          Size       "

W5      1-3, 4-6            ROM     32 Kbytes ROM

W6      1-3, 4-6            Size       "

W5      3-5, 4-6            ROM     64 Kbytes ROM

W6      3-5, 4-6            Size       "