XX   XX   XXXXX   XXXXXXX  XXXXXX   XX   XXXXX

XX   XX  XX   XX  XX       XX   XX   X  XX   XX

XX   XX  XX       XX       XX   XX      XX

XX   XX   XXXXX   XXXXXX   XXXXXX        XXXXX

XX   XX       XX  XX       XX XX             XX

XX   XX  XX   XX  XX       XX  XX       XX   XX

XXXXX    XXXXX   XXXXXXX  XX   XX       XXXXX

XX   XX   XXXXX   XX   XX  XX   XX   XXXXX   XX

XXX XXX  XX   XX  XXX  XX  XX   XX  XX   XX  XX

XXXXXXX  XX   XX  XXXX XX  XX   XX  XX   XX  XX

XX X XX  XXXXXXX  XXXXXXX  XX   XX  XXXXXXX  XX

XX   XX  XX   XX  XX XXXX  XX   XX  XX   XX  XX

XX   XX  XX   XX  XX  XXX  XX   XX  XX   XX  XX

XX   XX  XX   XX  XX   XX   XXXXX   XX   XX  XXXXXXX

NAVPROGseven

Aircraft Navigation & Flight Planning

for CP/M and the Heathkit/Zenith H-89

=====================================

(c) Copyright 1982 Alan Bose

Changes for CP/M and MBasic 5.21

(c) 1982 by Glen E.  Hassebrock, Jr.

Changes for MS-DOS and Hewlett Packard HP-150

(c) Copyright 1984 Alan Bose

1224 Allison Lane

Schaumburg, IL 60194

1

TABLE OF CONTENTS

=================

Introduction . . . . . . . . . . . . . . . . . . . . . . .  3

User-Supported Software . . . . . . . . . . . . . . .  6

Installation . . . . . . . . . . . . . . . . . . . . . . .  8

Start Running . . . . . . . . . . . . . . . . . . . .  8

Disc Layout . . . . . . . . . . . . . . . . . . . . .  9

Airport/Navaid Data Entry  . . . . . . . . . . . . . . . . 10

Update Mode . . . . . . . . . . . . . . . . . . . . . 13

Aircraft Data Entry . . . . . . . . . . . . . . . . . . .  14

Navigation -- Preparing a flight. . . . . . . . . . . . .  15

Automatic Route Preparation . . . . . . . . . . . . . . .  17

Pre-Planned Routes. . . . . . . . . . . . . . . . . . . .  18

Data File Listings. . . . . . . . . . . . . . . . . . . .  18

Program Specs . . . . . . . . . . . . . . . . . . . . . .  19

NAVPROGseven / Contents                                  Page 2

An Introduction to NAVPROGseven

Aircraft Navigation & Flight Planning

=====================================

(c) Copyright 1982, 1984 Alan Bose

1224 Allison Lane

Schaumburg, IL 60194

Past-President, Taildragger Flyers, Inc.

Chicago Rep, Aviation Safety Institute

NAVPROGseven is a series of nine programs designed to prepare

a flight log that is ready for use in the cockpit.   NAVPROGseven

stores  data about each aircraft you fly and each checkpoint  you

fly over, and saves that information for subsequent flights.

The features and functions of NAVPROGseven include:

1).  Easy input & revision of the airport/navaid data base.

2).  Two RNAV functions to provide latitude and longitude of

a location based on fixes taken from known points.

3).  Aircraft performance data stored for each of the planes

you fly.

4).  Easy  access  and display of checkpoint information  as

you  plan  your  route  of  flight.   Automatic  flight

planning  selects navaids closest to your great  circle

route  and  plots several  alternate  routings.   Often

flown routes can be stored for later use.

5).  Great  circle  navigation  between  checkpoints   using

aircraft performance data,  and printout of a ready-to-

use flight log.

6).  Climb/descent profiles based on aircraft performance data.

7).  Multiple  sort criteria to organize airport/navaid data

into easy-to-use printout.

The NAVPROGseven program uses the latitude and longitude  of

your  checkpoints  to calculate the heading required for a  great

circle  (the  shortest  distance  between  two  points  around  a

sphere).  The printed flight log tells you the field elevation at

your  departure  and  destination  points,   navaid   frequencies

enroute,  distances for each leg and  total  remaining,  true and

NAVPROGseven / Introduction                              Page 3

magnetic course, magnetic heading corrected for wind and magnetic

variation,  groundspeed corrected for climb and/or cruise & wind,

ETE  &  ETA  for each leg,  fuel usage based on cruise  or  climb

during the leg with startup/taxi/takeoff fuel accounted for, fuel

remaining, and a warning if reserves will be less than VFR or IFR

minimums.

In  addition a synopsis of the flight tells you  fuel  used,

reserves  in gallons and time,  fuel/time/distance used to climb,

and  how far from your destination you should begin your  descent

and how fast your descent should be to maintain a gentle 2 degree

descent gradient based on the groundspeed of your last leg.   The

descent profile assumes that you will be holding cruise  airspeed

during descent.

The  printed flight log has distance and true course of each

leg  conveniently  arranged  for easy entry  into  your  aircraft

navigation  computer,  if  used.   Or it provides a  very  useful

flight  log  as you cross-check with your  E6B  in  flight.   You

should  note that NAVPROGseven is designed for the  mid-latitudes

of  the  northern hemisphere over distances  normally  associated

with  light aircraft.   For European pilots,  most functions have

been designed to operate east and west of the Prime Meridian.

The  seed that started this series of programs  was  NAVPROG

version  14.4  by  Leland Young and his article on  great  circle

navigation in the February 1980 issue of Microcomputing Magazine.

Additional  source  material  includes  "Dutton's  Navigation   &

Piloting"  (U.S.    Naval  Institute,  Annapolis)  and  the  "VNR

Encyclopedia  of Mathematics" (Spherical  trigonometry,  Gellert/

Kustner/Hellwich/Kastner).

NAVPROGseven  was initially written & tested over a span  of

eighteen months and was introduced in early 1982 for the Heathkit

H-89  computer using the HDOS operating system.   In mid-1982  it

was  converted  to CP/M by Glen Hassebrock.   Both versions  have

been  in  use by hundreds of pilots who  purchased  the  original

programs  through the Heath Users' Group.   I am indebted to  the

many  pilots  who have offered their  comments,  suggestions  and

ideas over the years.

Though  technically  a  great  circle  route  would  involve

constantly  changing your heading during each  leg,  NAVPROGseven

gives you a single heading for each leg that closely approximates

a  great circle -- just as a single heading taken 'mid-leg'  from

your Sectional or WAC chart will also approximate a great circle.

(Your ruler's straight, it's the chart that's curved).

NAVPROGseven / Introduction                              Page 4

Needless  to  say,  it is vitally important that you  always

double  and  triple check your input.   And although  the  system

makes  planning a cross-country flight quicker  and  easier,  you

should always question and verify your output.  And it is nothing

more  than good navigating practice to cross-check your  position

at  frequent intervals during the flight -- don't go charging off

blindly, always question.

The  fate  of  the Air New  Zealand  747  guided  unerringly

towards  an  Antarctic mountain bears a sober reminder  that  the

most  technologically  advanced airplanes in the world are  still

subject to human error,  whim and fancy.  There is no replacement

for common sense, no matter what 'the computer' says.

Of course, it is recommended that you 'test fly' the program

before you put it to actual use.   Although it works well for the

planes  I fly,  you should make sure that the output you  get  is

within reason for your aircraft.  Needless to say, there are many

variables  to  a flight that this program simply  does  not,  and

cannot, take into account.

Alan Bose

1224 Allison Lane

Schaumburg, IL 60194

NAVPROGseven / Introduction                              Page 5

***********************************************************

*                 USER SUPPORTED SOFTWARE                 *

***********************************************************

* The   author   of  a  user-supported   program   freely *

* distributes his work,  and requests that users who find *

* the  program  useful and of value,  send the  author  a *

* suggested  contribution  (NAVPROGseven:  $45).   Unlike *

* conventional software, the user is able to evaluate the *

* software  fully before spending any money.   In  return *

* the  author  may  send  contributing  users  additional *

* material such as printed documentation, source code and *

* most  importantly,  notification of patches,  bugs  and *

* upgrades.    The   system   depends   on   the   active *

* participation of the user community, for without a flow *

* of  contributions for deserving programs,  authors will *

* soon abandon this worthwhile method of distribution.    *

*                                                         *

* Since  these programs are subject to unlimited  copying *

* and distribution the prospective user is hereby put  on *

* notice  that the programs may contain or develop faults *

��th� consequence� o� whic�  th� autho� canno� b� hel䠪

�����������responsible��  Th� prospectiv�  use� is��b� virtu�o� *

���������� obtainin� an� usin� thes� programs� assumin� ful�  ris� �

���������* fo� an� an� al� consequences.                           *

***********************************************************

* These  programs  were developed and tested  on  version *

��2.2.0��o� th� CP/� operatin� syste� an� an� Microsof� *

�����������Basi� versio� 5.21�� a� distribute� b� Heat� an�Zenit� *

���������* Data Systems.                                           *

*                                                         *

* The  author  cannot  predict  that  the  programs  will *

* perform   on   future   versions  of  CP/M  and  Basic. *

* However,  if a version dependent program surfaces,  the *

* author  may  make available to contributing  users  any *

* fixes and upgrades.   Individual attention to  problems *

* of this nature is  left to the sole discretion  of  the *

* author.                                                 *

***********************************************************

NAVPROGseven / Introduction                              Page 6

***********************************************************

* NAVPROGseven is a copyrighted program,  and is intended *

* for  your  private,   non-commercial  use  only.   USER *

* ALTERATIONS AND MODIFICATIONS MAY NOT BE RE-DISTRIBUTED *

* WITHOUT PRIOR WRITTEN PERMISSION FROM THE AUTHOR.  This *

* includes  conversions  of  the  programs  to operate on *

* other brands of computers.                              *

*                                                         *

* If you find this program a useful and valuable addition *

* to your software library, the suggested contribution is *

* $45  (US),  sent  to  Alan  Bose,  1224  Allison  Lane, *

* Schaumburg  IL 60143.   Contributing users will receive *

* the  20-page NAVPROGseven Users  Manual,  the  complete *

* source  listing,   notification  of  upgrades,  and  if *

* desired,  a  copy  of  the  source code in ASCII format *

* (include a blank disc with your check).                 *

*                                                         *

* Fresh  copies  of  the  distribution  software  may  be *

��obtaine�  b� sendin� � blan� dis� an� � self-addressed� *

���������* stampe� retur� maile� t� th� addres� above�             *

***********************************************************

NAVPROGseven / Introduction                              Page 7

INSTALLATION

============

��������NAVPROGseve i� designe䠠 t operat堠 o ᠠ dual-driv�

��������Heathkit/Zenit� H-8� compute� syste� wit� 64� o� memor� usin� th�

��������CP/͠ operatin� syste� versio� 2.2.03+�� an� Microsof� Basi�

��������versio� 5.21+��  Th� program� us� curso� addressin� sequence�

��������particula� t� th� Heathkit/Zenit� brand.

Format  a CP/M  disk  and  install on it a copy of Microsoft

Basic 5.21+.   This will be your program disk.  You may also make

this  disk  capable of cold booting by  including  the  operating

syste� (us� SYSGEN)��  I� thi� optio� i� chose� yo� ma� ge� th�

��������NAVPROǠ men� automaticall� o� cold-boo� b� settin� th� comman�

��������lin� MBASIà MENU/S:255/F:� (us� CONFIGUR)��  Th� /F:�� i� no�

��������necessary�� bu� allow� mor� memor� t� b� use� fo� dat� tha�

��������without it.

Initializ� � secon� dis� fo� data��  Us� PI� t� transfe� program�

��������and sample data to your discs.

RUNNING NAVPROGseven

====================

Place  your program disc on the A:  drive and your data disc

on the B:  drive.  When NAVPROGseven starts running it will check

that  the proper discs are accessable on the proper  drives.   If

not the program will stop and request them before continuing.

You  may run NAVPROGseven by:

1�� A� boot-u� automaticall� (i� auto-comman� lin� wa� se� usin�

��������    CONFIGUR for this option).

2)  At the CP/M A> prompt by typing   MBASIC MENU/S:255/F:2

��������3)  At the CP/M A> prompt by typing   SUBMIT NAVPROG

��������    (SUBMIT.COM must be on your program disc for this option).

Note that the carriage return is not required for any  'one-

key  responses'  such as <Y>es,  <N>o,  <E>ast,  <W>est  or  when

selecting numbers from the menu.

NAVPROGseven / Installation                           Page 8

When  you  have  completed the  installation  your  'working

discs' should contain the files shown below:

PROGRAM DISK (for use on drive A:)

(BIOS   .SYS)   your operating system (optional)

(SUBMIT .COM)      "        "

MBASIC  .COM    your Microsoft Basic 5.21+

DISKAID .DAT    disc identification

NAVPROG .SUB    CP/M SUBMIT file

MENU    .BAS    the main menu program

AIRINPUT.BAS    input/revise airport & navaid data

AIRCRAFT.BAS    input/revise aircraft data

AIRROUTE.BAS    manual route preparation

OLDROUTE.BAS    retrieve a stored flight routing

NAVPROG7.BAS    navigation & flight log

AUTONAV .BAS    automatic route selection

RNAVREF .BAS    navaid bearing cross checks

AIRALPHA.BAS    database listings

DATA DISK (for use on drive B:)

DISKBID .DAT    disc identification

README  .DOC    general documentation

NAVPROG .DOC    detailed User's Manual

N73116  .       sample aircraft data -- ERAse manually or

N81259  .       using program after you experiment

AIRINDEX.RND    { over 100 checkpoints already on file,

AIRPORTS.RND    {   can be revised from the program,

{   or ERAse these two & start over.

ROUTINGS.DAT    sample index of routes on file.

Update using program only.

BEH     .LUK    sample routes on file.

LUK     .BEH        Update using program only.

X06C    .1H2

X1H2    .06C

��������          etc...

NAVPROGseven / Installation                           Page 9

INPUTTING AND UPDATING AIRPORT/NAVAID DATA

==========================================

Note  that the carriage return is not required for any 'one-

key  responses'  such as <Y>es,  <N>o,  <E>ast,  <W>est  or  when

selecting numbers from the menu.

All facilities are identified by their FAA identifier, or if

one is not assigned,  feel free to make one up.   Identifiers are

usually 3 letters for navaids,  3 or 4 characters for airports, 2

letters  for  ILS compass locators,  and a  name  or  pseudo-word

(usually  5  letters)  for  airway  intersections  and  reporting

points.   NAVPROGseven  uses  up to 5 characters  for  checkpoint

identifiers.

You  don't  have  to  worry  about  duplicating  identifiers

because NAVPROGseven accepts them as normal.  For example, DPA is

the identifier for DuPage County Airport,  near Chicago,  but DPA

is  also  the identifier for DuPage Vortac located 4.3 nm to  the

west.   Since  the vortac is not at the airport I  made  separate

entries for each.  When asked for DPA the computer will find both

and  ask  which one you want,  the airport or the  navaid.   (See

notes on co-location below).

During new data input the computer will ask for the following:

FACILITY CODE:

What one would expect to find there.   You can make up  your

own but 'V' and 'N' (VORs & NDBs) have special significance since

you'll be asked to enter the frequency in the next step.

Notes about facility code:

Plotting  a  great-circle  route over  short  distances

(less  than a nautical mile or so) can exceed the limits  of

the  trigonometric functions available  under  Basic.   I've

talked  with  the  writers of other nav programs  for  other

micros,  and they've run into the same problem.  When you're

working on a global or continental scale,  1 or 2 miles from

checkpoint to checkpoint is very small indeed.

NAVPROGseven has a provision that seems to handle  this

dilema  quite nicely.   You'll notice that the Facility Code

field has space to enter 2 codes,  not just one -- what  the

FAA refers to as 'co-located' facilities.   In my database I

have  used  'AV'  to indicate a VOR located at  or  near  an

airport, or 'AN' for an airport & NDB, etc.

If the navaid is within 3 or 4 miles of the airport,  I

still consider them  co-located, and enter the  latitude and

NAVPROGseven / Airport & Navaid Data                     Page 10

longitude  of  the NAVAID.   At that  distance  the  airport

should  be  in  sight when flying VFR.   On an  IFR  flight,

approach  plates  would be in use for  this  final  approach

phase.   In  either case,  you shouldn't be looking at  your

flight log.

FREQUENCY:

If  you entered a 'V' or 'N' above,  you will then be  asked

the  frequency  of the navaid -- otherwise this step is  skipped.

If  the  facility  is  not  a navaid  but  you  want  to  have  a

communications frequency listed, you can enter it later using the

update  mode,  though  I find it less confusing to know that  all

frequencies shown on the printout are NAV frequencies rather than

mixing NAV & COM.

NAME:

Whatever  you would like to call the checkpoint,  navaid  or

airport.  Be sure, however to include the state or province since

the  sorting  program  will  separate that out  of  the  name  to

organize all of your data into a nice, neat printout by state.

The  state  must be preceded by a comma,  or a comma  and  a

space,  such as ',  MI' for Michigan or ',IL' for Illinois.  I've

chosen  to  use  the postal code  abbreviations  because  they're

short,  but AR is Arkansas,  not Arizona which is AZ; and I don't

know if I'll ever keep Michigan (MI),  Missouri (MO), Mississippi

(MS),  Minnesota (MN) and Monatana (MT) straight,  not to mention

Massachusetts (MA),  Maine (ME), and Maryland (MD).  The code you

use doesn't matter as long as you're consistant.

LATITUDE & LONGITUDE:

Take   this   from   your   charts,   the   Airport/Facility

Directories,  or AOPA's Airports USA.  Or you can use the pseudo-

RNAV  function  to  calculate the latitude and  longitude  of  an

intersection or a landmark based on the distance and bearing from

a navaid you already have on file,  or based on the bearings from

two navaids (like you do in the air).

When entering longitude you will be prompted to specify east

or west longitude.   Since west longitude is the  default,  North

American  pilots  may  simply hit a  carriage  return.   European

pilots should specify east or west longitude as required.

NAVPROGseven / Airport & Navaid Data                     Page 11

When  using  the RNAV functions to  calculate  latitude  and

longitude,  make sure the navaid(s) are in range.  When using two

navaids  you should remember basic navigation procedures:  if you

draw a line between the two navaids,  the closer you are to  that

line the worse your accurracy.   And if you are on the line it is

impossible  to  determine your position!  It is best if  the  two

navaids are about 30-60 degrees (or 120-150 degrees) apart.

The  accuracy of the RNAV functions can only be as  accurate

as  your  charts and the measurements you take.   Since  magnetic

variation changes from year to year,  it is best if you use  true

bearings  for entry.   With care the typical RNAV error should be

less  than one nautical mile.   Use the latitude  and  longitudes

published  in  the Airport/Facility Directory whenever  possible.

Also,  it's  wise  not  to use a point calculated  with  RNAV  to

calculate  another  point  with  RNAV  as  any  errors  will   be

cumulative.

Notes on RNAV:

The pseudo-RNAV calculations with bearings  approaching

due  east or due west can strain Basic's accuracy as several

trigonometric functions approach infinity.   While  normally

still  within the tolerance of VOR equipment,  you should be

aware  that  such  discrepancies  can  exist  under  certain

conditions,  since  NAVPROG may err in one  direction  while

your VOR may err in the other.

These  cross-bearings  are  intended  to  be  a   handy

reference  for the VFR pilot who doesn't have air-borne RNAV

equipment.   If you have onboard RNAV capability you  should

ALWAYS  enter  waypoints into your equipment from  published

sources.

European pilots:

The  RNAV  functions mentioned here are  not  currently

supported east of the Prime Meridian.

MAGNETIC VARIATION:

Read  straight  from your chart.   Be  aware  that  magnetic

variation changes slightly from year to year.   The flux goes one

way  then the other over a cycle of about 10-15  years.   Science

has  several  theories  why the earth's magnetic field  seems  to

wander,  but  the important thing to know is that small  discrep-

ancies  in the magnetic course and magnetic heading  calculations

will become evident over a period of time.   The only solution is

to update the magnetic variation for each entry in your  database

on a periodic basis.  RNAV calculations are also affected by this

constant shifting of magnetic variation.

NAVPROGseven / Airport & Navaid Data                     Page 12

ELEVATION:

Right from your chart if it's an airport.  Navaid elevations

can  be found in the Airport/Facility Directory.   Elevations  at

other locations can be approximated from your charts.

Notes on elevation:

The primary use for the elevation in the program is for

airports.   It  is used to calculate your departure climb to

altitude  and the descent to your destination  airport.   If

you  choose  to enter the elevation of  navaids  (which  are

given  in  the Airport/Facility Directory) the program  will

give  a  cursory  check to see  that  your  selected  cruise

altitude and climb/descent profile will clear the navaid.

UPDATE MODE

===========

In  the UPDATE mode you can revise the data on file as often

as you like.  You can delete unneeded entries or let your library

of checkpoints continually grow.

In the update mode entering a 'D' for the Ident will  delete

the entire entry -- erasing its subsector from the index & master

file.  Later, when you add new data, the program searches for any

'holes'  left by a deletion and fills them in first.   If a  hole

can't be found, the new data is added to the end of the file.  To

maximize disc space it is best you delete unwanted entries before

adding new ones.

Of  course  airports do close,  navaids are  decommissioned,

moved,  renamed or frequencies re-assigned.   NOTAMS (Notices  to

Airmen)  are  an  excellent way to assure that your  database  is

current and up-to-date.

NOTAMS are published every 14 days and subscriptions are

available from the

U.S.  Government Printing Office,

Washington, DC 20402.

The Aiport/Facility Directories are published every eight weeks

by the National Oceanic and Atmospheric Administration

National Ocean Survey

Riverdale, MD 20840.

AOPA's Airports USA is published yearly for members of the

Aircraft Owners & Pilots Association

421 Aviation Way

Frederick, MD 21701

NAVPROGseven / Airport & Navaid Data                     Page 13

AIRCRAFT DATA

=============

When  you  enter your aircaft N-number the  the  performance

data  is  read from the file,  or if it can't be found you'll  be

asked  to enter data from your Aircraft Operations  Manual.   The

program  takes into account fuel used  for  startup/taxi/takeoff,

fuel/time/distance to climb to your normal cruise altitude,  fuel

comsumption,  fuel-on-board, true airspeed, and the cost per hour

to operate the plane.

When  entering your 'standard' cruise performance you  might

consider that optimum altitude for a normally aspirated engine at

75%  power is in the 6-8000 foot range;  giving the highest  true

airspeed  for  fuel burned.   As the power setting  is  cut,  the

'optimum  altitude' increases.   In actual flight,  however,  the

winds aloft and the time & fuel used to climb there will probably

effect efficiency more than optimum altitude.  Also remember that

flight  at 75% power uses an average of 13% more fuel for only  a

6% increase in speed over a flight at 65% power,  while flying at

55%  power  uses 25% less fuel with only a 12%  speed  loss  when

compared to a flight at 75% power.

It  should  be  noted  that in  calculating  your  climb  to

altitude  NAVPROGseven uses a straight line average based on  the

standards  you enter here about your  aircraft.   Long,  extended

climbs  at gross weight to higher altitudes than your  'standard'

entered  here may consume significant amounts of additional fuel.

But I'm not telling you anything new,  huh? Consult your Aircraft

Operation Manual, please.

Also,  the  'standard cruise' speed & fuel  consumption  you

enter  here  is  guaranteed to vary on each flight  depending  on

altitude,   temperature,   etc.   However  when  each  flight  is

calculated you will have the opportunity to enter  'non-standard'

cruise parameters that will be used for that given flight.

Every plane is different,  and your Operations Manual is the

only  authoritative source for this kind of  information.   Sure,

general  rules-of-thumb could have been included in the  program,

but  for  your plane would you want a rule-of-thumb  (abbreviated

R.O.T.) calculated to sixteen decimal places?

NAVPROGseven / Aircraft Data                             Page 14

NAVIGATION

==========

Using  the  program you simply enter the  number  of  check-

points  you'll  be using,  then the identifiers for those  check-

points.  If the route is one that you expect to fly again you can

save  it for future use.   Note that the checkpoint data  is  not

saved  but  rather index numbers that allow the computer to  find

the  data  in the main file at a later date.   This  way  if  you

update  checkpoint data,  the next time you use the stored flight

the new data will automatically be picked up.   Of course,  if in

the meantime you've deleted one of the checkpoints,  the  program

will abort and return you to the menu.   Regardless,  you will be

asked  to  verify  that  the route of flight  is  correct  before

calculations begin.

Next  you would enter your aircraft's N-number,  the  Flight

Service Station you'll close your flight plan with, and the winds

at  each  checkpoint.   Of  course winds at  your  departure  and

destination should be surface winds,  while winds aloft should be

used enroute.  Surface winds and winds aloft may be obtained from

your FSS,  the NWS,  or thru an online service such as CompuServe

or others.  To enter no-wind for a checkpoint you can enter '0,0'

(or  a  comma by itself followed by a carriage return  gives  the

same result).

The computer will then ask for your cruising altitude,  true

airspeed,  fuel consumption & fuel-on-board.   The computer  will

show the 'standard' situation you entered with the aircraft data,

and  by  simply  hitting  a carriage return  for  each  of  these

questions  the standard data will be used,  or you can deviate by

entering  new data for the flight.   Note that the computer  does

not  adjust  your  true airspeed or fuel  rate  when  you  change

altitude,  nor does it take into account outside air temperature.

While this could be done using some general rules-of-thumb,  it's

best  that you refer to the Operations Manual for your plane  and

enter either the book values or values based on your experience.

After  you enter your cruise altitude the computer uses  the

departure  elevation  to calculate your climb  profile,  and  the

destination  elevation to figure the descent profile.   Note that

during  your  climb to altitude the groundspeed printout  is  the

average  for  the  entire leg,  taking into  account  your  climb

groundspeed (if you haven't reached altitude), cruise groundspeed

(if  you reach altitude before the checkpoint),  and  wind.   The

climb airspeed is an average based on the normal climb data,  and

CAS  is  based on your altitude on a  standard  day.   These  are

guidelines,  not  hard  numbers  -- double check  the  Operations

Manual for your plane.

NAVPROGseven / Navigation                                Page 15

The  computer  then checks your cruising altitude  with  the

elevation for each of your checkpoints enroute.   If your  cruise

altitude  brings  you less than 1,000 feet AGL at any  checkpoint

the computer will caution you.   Just because the computer clears

this check DO NOT ASSUME THAT THIS IS A SAFE CRUISING ALTITUDE!!!

The  computer is only checking SURFACE ELEVATIONS,  ONLY  AT  THE

CHECKPOINTS YOU SPECIFIED, and ONLY AT THE CHECKPOINT ITSELF, NOT

IN  BETWEEN.   The machine has no way of knowing if the elevation

you  originally  entered  was correct.   It also has  no  way  of

knowing  that between the two checkpoints there is a 14,000  foot

mountain  and  that  there's  a broadcast tower  on  top  of  the

mountain!  CHECK YOUR CHARTS and know your clouds -- steer  clear

of cumulus granite and stratus erectus!

In  the  fuel column a single hashmark will appear  if  fuel

reserves  are  less  than  45 minutes (below IFR  and  VFR  night

minimums),  and  a  double hashmark signifies that less  than  30

minutes  of fuel remain (below VFR minimums).   If the  hashmarks

appear you should plan to make an earlier fuel stop.

It's  usually inefficient to climb more than 10 minutes  for

each  hour  of estimated time enroute unless good  tailwinds  and

high  altitude cruising efficiency offset the fuel and time  used

to  climb.   With NAVPROGseven you can cycle thru the  flight  as

many  times as you wish,  plugging in new variables and  printing

out  the  best  flight  profiles  at the end  of  each  cycle  --

comparing fuel vs.  time vs. dollars in order to get the best use

out of your airplane.

NAVPROGseven / Navigation                                Page 16

AUTOMATIC ROUTE PREPARATION

===========================

Enter your point of departure and your destination and  this

program  scans the database,  selecting the checkpoints along the

way that most closely follow a great circle route.   The  program

automatically  prepares  several  alternate routings for  you  to

choose  from,  and with each pass the checkpoints are closer  and

closer together.

If  needed you can enter a specific checkpoint to  be  flown

over  (located to one side of a restricted airspace,  let's  say)

and  the computer will first route you to that checkpoint  before

turning  you towards your destination.   The sharper the  dog-leg

however,  the  longer  the time required to calculate  the  route

since more of the database must be repeatedly scanned.

This program will not tell you the 'best' route to fly,  but

it   will  give  you  several  logical  alternatives.    Military

Operation Areas,  large bodies of water,  restricted & prohibited

airspace are items the computer has no knowledge of.  If your 172

gets intecepted by an F-16,  don't tell them,  "NAVPROG said this

was the best route to fly",  and expect to get off!  On the other

hand,   NAVPROGseven  is  being  used  by  several  Coast   Guard

helicopter  crews around the country,  and I actually got a  call

one day at work from an Air Force general in Washington wanting a

couple enhancements....   so who knows.

The  first pass through the database will give you the  most

direct  route  -- the  fewest  checkpoints and  the  longest  leg

lengths.    Each  subsequent  pass  will  supply  more  and  more

intermediate  checkpoints  from your database  with  shorter  and

shorter  leg  lengths,  all the while trying to stay as close  as

possible to the great circle between the last checkpoint and your

destination.

The  more checkpoints,  the more zigs and  zags.   From  the

possible  routes,  you must take into consideration the reception

range of your navigation equipment,  which is of course  affected

by your cruising altitude,  station power, obstructions, etc, and

how  direct  you  wish  to fly  given  these  parameters  without

investing in an inertial navigation system for your Cub.

NAVPROGseven / Auto Route                                Page 17

NAVIGATING A PRE-PLANNED ROUTE

==============================

This program retrieves the checkpoints for a flight that was

saved  at  an  earlier date.   If the checkpoint  data  has  been

revised  the new data is automatically used.   If the  checkpoint

data  has  been deleted since the last time the route was  flown,

the  program will abort and return you to the menu.   A  printout

can  be obtained of all routes on file including all RNAV  cross-

references.

SORT AND PRINT DATA ON FILE

===========================

The  sort  program  uses a Schell-Metzner  sort  routine  to

arrange   the  airport/navaid  data  on  file  into   easy-to-use

printouts.  The data on file can be printed out by Identifier; by

State & Ident;  by State & City;  by State,  Facility & Ident; or

unsorted straight from the file.

NAVPROGseven / Pre-Planned Routes                        Page 18

PROGRAM SPECS AND NOTES

=======================

Data  for each airport,  navaid,  facility &  checkpoint  is

stored on random file AIRPORTS.RND,  five to a sector.   A random

index,   AIRINDEX.RND,   holds  just  the  identifiers  for  each

checkpoint  and tells the computer the relative position of  each

sub-sector  of  data  in  the  main  file.    RNAVLIST.DAT  is  a

sequential file that saves the fixes you use in the RNAV mode.

The primary database is contained in these three files which

are  constructed  and  maintained  by  AIRINPUT.BAS  (RNAVREF.BAS

automatically  updates  & maintains the RNAVLIST.DAT  file  after

each flight).

For maximum speed in retrieving airport and navaid data, the

entire  index is read into memory rather than doing repeated disc

accesses��  However�� i� yo� hav� � 64� machin� an� ad� a lo�o�

��������airports�� navaids�� et� t� th� file� yo� ma� ru� ou� o� memor�

��������runnin� NAVPROG7.BAS��  I� thi� occur� yo� shoul� conside� eithe�

��������mor� memor� o� breakin� u� you� dat� int� multipl� discs��  Se�

��������below.

It  should be noted that NAVPROGseven was designed  for  the

general  aviation  pilot.   This is not to say that you  have  to

stick  to  one  region of the country or stay within  a  thousand

miles of home.  No way.  But by the same token, you're not Pan Am

and  this isn't an HP-3000.   NAVPROGseven has an upper limit  of

5400 miles for any given great circle computation.

Since most flights tend to radiate out from one's home base,

some pilots have chosen to set up separate data discs for flights

east  and west (or north and south) of home.   Normally this  was

done because earlier machines only had 64K of memory and couldn't

hold the entire index at once. However, if you decide to do this:

1)   make  sure that all data files including your aircraft  data

and file DISKBID.DAT are copied over to your new data disc.

2)   if divided between north & south data,  provide some overlap

to  the north on your 'Southern' disc.   Great circle routes

north  of  the equator are always bowed  towards  the  north

pole.

3)   as   you   edit  your  database,   always  delete   unneeded

checkpoints  before  adding  new  ones  to  make  the   most

efficient use of disc and memory space.

NAVPROGseven / Program Specs                             Page 19

AIRCRAFT.BAS takes data for each aircraft you fly and stores

it  in separate sequential files with the plane's N-number as the

filename.   Two aircraft,  N73116 and N81259,  are on file,  with

which you can experiment.  ROUTINGS.DAT is an index of routes you

have  saved  for later use -- it is created by  AIRROUTE.BAS  and

called  by  OLDROUTE.BAS.   The individual routes are  stored  in

separate sequential files;  i.e., information about a flight from

Benton Harbor to Cincinnati Municipal (Lunken Field) is stored in

file BEH.LUK.

A file FLIGHT.SEQ is created temporarily by AIRROUTE.BAS  or

OLDROUTE.BAS  to pass the file locations of the current route  of

flight  to NAVPROG7.BAS and then to RNAVREF.BAS after which it is

erased.   RNAVREF.BAS  looks at the route of flight and  searches

RNAVLIST.DAT  for  any checkpoints that were entered  using  RNAV

cross-bearings, if so they are printed on your flight log for in-

flight reference.

Why doesn't NAVPROGseven generate my FAA flight plan for me?

A good question.  In the very first, experimental and unpublished

versions of this program I included just such a routine.   Rather

simple  actually,  but why stop at a flight plan?  Why  not  dump

right  into the autopilot and we can all fly around like so  many

locomotives  on  rails?  My feeling is that preparing  one's  own

flight plan form gives you time to pause,  reflect and review the

upcoming flight.  I love computers, but it is the captain who has

the final word.  Let the computer do the number-crunching it does

so well, but don't let it become the de-facto pilot-in-command of

your plane as well.

NAVPROGseven / Program Specs                             Page 20

#####################################################################################