EPROM

- check a bit for monitor active flag

if monitor active is true 		->  long call monitor

else

stuff a long return vector into location for INT2

(absolute location 8) which initializes the NMI

call.  This vector points to code:

-> load the sp from 20000 - constant

(to get a new register save area)

-> swap register halves and push (for 68K inversion)

-> halt

** processor is now idle awaiting another nmi **

system call:

INT21			; go to location 084h

JMP	>6000		; yet to be determined

MOVE STACK 20000H

PUSHA

SETINT

HALT

** WAITING FOR NMI **

WHEN ONE COMES GO TO LOCATION 8 WHICH HAS A:

RETI			;LONG RETURN FFROM INTERRUPT

THIS LONG RETURN GOES TO CODE WHICH:

POP A

RETI

TRINITY SYSTEM MEMORY MAP:

FF0000 - FFFFFF			68000 MEMORY MAPPED I/O SPACE

FE0000 - FEFFFF			68451 MMU SPACE

FD0000 - FDFFFF			68000 EPROM SPACE

F00000 - FCFFFF			Z80 SLAVES

800000 - EFFFFF			80188 SLAVES

700000 - 7FFFFF			MEMORY MAPPED BIT GRAPHICS BOARD

080000 - 1FFFFF			MAXIMUM UNIX SYSTEM V SYSTEM RAM

030000 - 07FFFF			UNIX SYSTEM RAM  (SYSTEMS WITH 512K)

000400 - 02FFFF			UNIX KERNEL

000000 - 0003FF			68000 INTERRUPT VECTORS

Z80 SLAVE MEMORY MAP:

F000 - FFFF			EMULATOR

0100 - EFFF			USER

0000 - 00FF			SYSTEM

80188 SLAVE MEMORY MAP:

FFE00 - FFFFF			EPROM

80000 - FFDFF			RESERVED FOR FUTURE EXPANSION / GRAPHICS

01600 - 7FFFF			USER RAM (512K SYSTEM)

00600 - 015FF			DOS EMULATOR

00500 - 005FF			MSDOS COMUNICATIONS AREA (ALA IBM)

00400 - 004FF			ROM COMMUNICATIONS AREA (ALA IBM)

00000 - 003FF			INTERRUPT VECTORS

6/12/84

BJG

######################