DUALPORT.DOC

Using Kaypro's to demo software

02/22/84  D. Berg

Recently  I was asked if there was a way to set up  multiple

displays using Kaypro computers at the user group meetings.   One

computer  was to run a software demo and the other units were  to

simply display a copy of what was on the main unit.  After giving

the problem a little thought this is what I came up with.

First  a program would be needed in the main "demo" unit  to

catch  the  characters going to the screen and pass them  to  the

serial  port  where the remote displays could  read  them.   This

program would need to fit in the machine in a manner such that it

would  not  interfere  with normal operations - but  would  catch

everything going to the screen.   Then a cable(s) would be needed

to connect the main unit with the display units.  And finally the

display  units would need to have a terminal program  running  in

them - like TERM.COM.

Operating  procedures  for the setup above are  as  follows:

The  computers are set-up and connected with the cable (described

later).   The  baud rate would be set to 9600 baud and  the  TERM

program run.   These units will now function as display monitors.

The  main-demo unit is booted and the DUALPORT program  run.   It

sets  the unit to 9600 baud and patches the I/O to go to both the

screen and the serial port.  DUALPORT will remain in effect until

the system is powered off or the reset button is pressed.  CTRL-C

will not affect the operation however.

The cable needed in this setup is simple requiring only  two

wires  be hooked up between the units.   Refering to the  diagram

below  the HOST unit is the main demo unit and the REMOTE unit is

the  display units.   Multiple display units can be connected  in

parallel  qith each other.   There is a limit to the number  that

will work but it should handle up to 6 units.   On all connectors

pins 5,  6,  and 8 are connected together on that connector only.

Pin 7 of all connectors are connected together in parallel.   Pin

2 of the HOST connector is connected to all pin 3's of the remote

connectors.   This  forms a one-way communications link from  the

HOST to all the REMOTE units.

HOST                           REMOTE

--------                        --------

1                                1

2--------------+                 2

3              +---------+-------3

4                        !       4

*---5                        !       5---*

*---6                        !       6---*

!   7--------------------+-- ! ------7   !

*---8                    !   !       8---*

V   V

To other REMOTE

display units.

########