The Z80MR creates two output files from an assembly language source file

with a .AZM extention. The assembly language file can be any combination

of Z80 mnemonics or assembler commands and pseudo-ops in the correct form

as described in the file Z80MR.DOC. Each line of the source program is

seperated by a carriage return linefeed and can be created on any editor.

The object file is the executable machine code represented by ascii

characters in a form known as Intel Hex Format. The object file will

have the same filename as the source file but will have a .HEX extension.

This format must be translated into the actual machine executable

binary values in order to be run on a computer. This can be done with

LOAD.COM or DDT.COM which came with your CP/M. (described later). Many

EPROM burners only except the program to be burned in Hex format.

The listing file is the other ouput by the assembler. It is ascii

representation of the code actually produced by the assembler. There

are many options available for controlling what is included in the

listing file as described in Z80MR.DOC. The listing file will have the

same filename as the source file but will have a .PRN extension.

Invoking the Assembler.....................................................

To assemble the file TEST.AZM the following command can be used:

Z80MR TEST <carriage return>

The assembler will read in the file TEST.AZM from the current drive and

create an object file TEST.HEX and a listing file TEST.PRN.

Controlling Output.........................................................

Either of the output files (or both) may be omitted. In addition, the files

may be located on different drives. Since the source file must always have

a .AZM extention we may use the extention to control the output.

Z80MR TEST.sol

The letter in the 's' position tells the assembler where to find the source

file and may be any disk drive letter from A to D. The  letter in the 'o'

position tells the assembler where to locate the object file and may be

A-D as above or Z if the file is not desired. The letter in the 'l' position

tells the assembler what to do with the listing file. It can be any drive

letter from A-D or Z if the file is not desired. In addition the listing

file may be sent to the console if an X is in that position.

Loading Hex Files..........................................................

The object file must be translated from Hex format to machine executable

values by another program. If the program was written to ORG at 100H the

program may be loaded with the CP/M utility LOAD.COM. All of the programs

on this disk with a .COM extension are ORGed at 100H. This is where CPM

loads its transient programs. To load TEST.HEX with LOAD.COM just enter

LOAD	TEST

The extention of .HEX is assumed.

If you have a program that does not ORG at 100H you must use DDT to read

in the .HEX file which it does by translating the Hex format to binary

as it reads it into memory. Once in memory you may manipulate it with

DDT even SAVE an image of the program in memory actually creating a .COM

file. If you write a program that ORGs where CP/M resides you can wipe

out CP/M and have to reset the computer to recover. See the file PHASE.DOC

for some more information on ORGing files at High memory locations.

:::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::

##############################################################################################################################