Table of Contents

0.1.  Deleted Commands                                                  1

0.1.1.  SET EIGHT-BIT-QUOTING                                           1

0.1.2.  SET FILE-BYTE-SIZE                                              1

0.1.3.  SET IBM                                                         1

0.1.4.  SHOW EIGHT-BIT-QUOTING                                          1

0.2.  New commands                                                      1

0.2.1.  LOG                                                             1

0.2.2.  SET SWAP-KEYS-BS&DEL                                            2

0.2.3.  SET PRINTER                                                     2

0.2.4.  SET FLOW                                                        2

0.2.5.  SET DISPLAY 80-COLUMN                                           2

0.2.6.  SET BAUD                                                        2

0.2.7.  SET PREFIX                                                      2

0.2.8.  SHOW PREFIX                                                     3

0.2.9.  SET DEFAULT-DISK                                                3

0.2.10.  SET TIMER                                                      3

0.2.11.  SERVER                                                         3

0.3.  Syntax of the commands                                            3

0.3.1.  LOG filespec                                                    3

0.3.2.  SET SWAP-KEYS-BS&DEL {on/off}                                   3

0.3.3.  SET PRINTER {on/off/slot number}                                3

0.3.4.  SET FLOW {off/xon/delay number}                                 3

0.3.5.  ESCAPE {p/c/s/?/0/b/d/ESCAPE}                                   4

0.3.6.  SET DISPLAY 80-COLUMN number                                    4

0.3.7.  SET BAUD number                                                 4

0.3.8.  SET PREFIX path                                                 4

0.3.9.  SHOW PREFIX                                                     4

0.3.10.  SET DEFAULT-DISK {slot/drive/volume} number                    4

0.3.11.  SET TIMER {on/off}                                             4

0.3.12.  SERVER                                                         4

0.4.  Vt52 emulation                                                    5

0.5.  Problems with loosing characters.                                 6

0.6.  Prodos errors                                                     6

0.7.  Com card drivers                                                  7

0.7.1.  Documentation for the vector area                               7

0.8.  Loading kermit 3                                                  9

0.9.  Problems                                                          9

0.9.1.  Installation                                                    9

0.9.2.  Usage                                                          10

0.10.  Caveat                                                          10

UPDATES FOR APPLE DOS KERMIT

0.1.  Deleted Commands

Since only text files have 7 significant bits/byte, several commands  have

been tied to the file type and have been deleted.

0.1.1.  SET EIGHT-BIT-QUOTING

Eight bit quoting is used for non text files  only  if  the  other  kermit

agrees.  Thanks to Rhoda Lichy for pointing this problem out.

0.1.2.  SET FILE-BYTE-SIZE

0.1.3.  SET IBM

Not all ibm controllers use mark parity. Therefor if you are connecting to

an ibm host determine the parity.  Then  "SET  PARITY  appropiately"  and  "SET

LOCAL-ECHO ON".

Here is what one site does to use kermit for file xfer:

We have an IBM 3033 and 4381 and use  both  3705/3725  and  7171/Series  1

front  ends.   The  only  difference  in front ends as far as any microcomputer

Kermit is concerned is duplex (local-echo on for the 3705, local-echo  off  for

the 7171).  In Kermit-65 they need to set the following parameters:

BAUD              - whatever is supported.

PARITY            - EVEN for us, maybe MARK for others.

(It is the PARITY which usually causes people problems.)

FLOW              - XON for the 7171, NONE for the 3705.

FLOW DELAY        - 00

The only software parameters that should cause them any problems are  BAUD  and

PARITY.  All the other things take care of themselves.

Thanks to Mark Johnson for his help in getting this straightened out.

0.1.4.  SHOW EIGHT-BIT-QUOTING

0.2.  New commands

These new commands have been added:

0.2.1.  LOG

When connected to a remote site, log the remote sessions printing  to  the

specified  file.  The  file  type and file warning protocols are observed. This

command is dependent upon the flow control  (xon&xoff)  working.  Without  flow

control  there  is little possibility of getting a correct copy of the terminal

session.  The loging begins when you connect to the remote  and  is  terminated

when  you escape back to the local kermit with the ESCAPE character followed by

1

UPDATES FOR APPLE DOS KERMIT

the "C" command.

0.2.2.  SET SWAP-KEYS-BS&DEL

Ever wish the del key was the back-space key. All this does  is  swap  the

delete & back-space key. Now when you are connected remotely you can choose the

key that is the correct place for your typing.

0.2.3.  SET PRINTER

This allows one to turn the printer on for printing what is  displayed  on

the  screen.  With all the different printers and printer cards there will be a

lot of variability here but flow control (xon&xoff) is probably  required  when

you  are connected to a remote site. The printer can also be toggled on/off via

the ESCAPE character followed by the command "P"

0.2.4.  SET FLOW

This allows one to use the xon&xoff protocol when you are connected  to  a

remote site. Delay timing is part of this command. Setting the timing will have

to  be  set  by experience.  Perhaps the best way to set the timing is to bring

the value down until you get failures and then double the timing figure.   Both

LOG and SET PRINTER will probably depend on flow control.

0.2.5.  SET DISPLAY 80-COLUMN

This allows one to specify a 80 column display. Several commands  are  re-

quired for the 80 column display to work correctly:

$0a - line feed, move cursor down one line

$0b - vt, clear screen from cursor down

$0c - form feed, clear screen and position to top left

$16 - syn, reverse scroll the screen

- this command necessary only for vt52 emulation

$1c - nak, move cursor right one

$1d - gs, clear line from cursor to the right

$1e,ch,cv - 3 chs in a row position cursor to ch & cv

- ch & cv each have a bias of 32

0.2.6.  SET BAUD

This command allows you to set the baud rate of your com card if  the  com

card allows it.

0.2.7.  SET PREFIX

This command is useful only when using prodos and allows one  to  set  the

prodos prefix.

2

UPDATES FOR APPLE DOS KERMIT

0.2.8.  SHOW PREFIX

Another prodos command to display the current prodos prefix.

0.2.9.  SET DEFAULT-DISK

This command now has the VOLUME option also.

0.2.10.  SET TIMER

This command allows KERMIT-65 to timeout on receive for file transfers.

0.2.11.  SERVER

This command allows KERMIT-65 to become  a  file  server.   Currently  the

server  functions  are  very  limited but will handle the remote "get" "send" &

"fin" commands. There are probably variants of these which will also work.  You

can  get  out  of  server mode via the "fin" or a ^c (thats control-c) from the

keyboard when not doing file transfers.  Because apple  requires  knowledge  of

file  types and there is no current way for the remote to change the file type.

You must set the appropriate file type before going into server mode.

0.3.  Syntax of the commands

0.3.1.  LOG filespec

Filespec is the name of  a  file  on  the  micro  that  will  receive  the

characters received from the remote. All the characters from the remote will be

recorded in the file while vt52 emulation is on, if vt52 emulation is off carr-

age return followed by line feed will be reduced to a carrage return.

0.3.2.  SET SWAP-KEYS-BS&DEL {on/off}

ON - swap the delete & back-space keys - when connected

OFF - delete & back-space keys have their normal codes

0.3.3.  SET PRINTER {on/off/slot number}

ON - turn the printer on, slot number is required

OFF - turn the printer off

SLOT number - printer card is in slot "number"

0.3.4.  SET FLOW {off/xon/delay number}

OFF - turn off flow control

XON - turn on xon&xoff flow control with the remote

DELAY number - delay the micro til xoff takes effect

Delay followed by number(including 0) delays the program for  number  mil-

liseconds  after the xoff is given to the remote. This delay allows the xoff to

take effect before the program continues.

3

UPDATES FOR APPLE DOS KERMIT

0.3.5.  ESCAPE {p/c/s/?/0/b/d/ESCAPE}

While connected to the remote the ESCAPE character followed by one of  the

following produces:

? - display this list of options

B - transmit a break signal to the remote

C - close the connection with the remote and return to micro

D - drop the phone line to the remote

P - toggle the printer on/off

S - give status of current connect

0 - transmit a null to the remote

ESCAPE - transmit the ESCAPE character to the remote

0.3.6.  SET DISPLAY 80-COLUMN number

Number is the slot the 80 column card is in.

0.3.7.  SET BAUD number

Number is the baud rate for your com card. For the super serial it can  be

300  to  19200.  Notice the com card is handled from within kermit for most ev-

erything we can think of.

0.3.8.  SET PREFIX path

Path is the prefix name for prodos. This string is prepended  to  all  fi-

lenames  and usually contains the volume name and directories which lead to the

filename.

0.3.9.  SHOW PREFIX

This command displays the current prodos prefix.

0.3.10.  SET DEFAULT-DISK {slot/drive/volume} number

Volume is an added option. Specify it as a decimal number.  Note  the  di-

splay from SHOW DEFAULT-DISK is in hex. So volume 255 would display as "FF".

0.3.11.  SET TIMER {on/off}

Turn the receive file transfer timeout on or off.

0.3.12.  SERVER

Enter file server mode for a remote kermit.

4

UPDATES FOR APPLE DOS KERMIT

0.4.  Vt52 emulation

Vt52 emulation has been reworked and should work much better,though  slow-

er.  One thing that is required is your 80 column card must handle the $16 com-

mand in order for reverse scrolling to work. The //e 80 col handles this fine.

The vt52 keypad has been defined using the open/closed apple.  For  II  or

II+ one will have to have a game paddle or joy stick and use the buttons.  When

a button/open/closed apple is pushed then the keys starting with 6,7,8 & 9 form

the top of the keypad. Key 6 is the blue key key 7 is the red key etc. The keys

directly below the 6,7,8 & 9 and shifted one-half key to the right form the se-

cond  row  of the keypad etc. Every thing is fine until you get to the last row

on the keypad. There the sp bar is bline and -> is  select  and  <-  is  enter.

Depending on your keyboard the -> & <- keys may not be in a very logical place.

With  those  two  exceptions  the  keypad is physicly similar to a vt52 keypad.

Remember the open/closed apple or the game button must be pushed (like the cntl

key) to get the keypad emulation.  How about a picture to clear  up  the  ques-

tions?

------- ------- ------- -------

|       |       |delline|   ^   |    ^ is up arrow

|   6   |   7   |   8   |   9   |

| gold  | help  |undelln|replace|

------- ------- ------- -------

------- ------- ------- -------

| page  |findnxt|delword|   v   |     v is down arrow

|   Y   |   U   |   I   |   O   |

|command| find  |undelwd|section|

------- ------- ------- -------

------- ------- ------- -------

|advanc|backup |delchar|   ->  |

|  H   |   J   |   K   |   L   |

|bottom|  top  |undelch|specins|

------- ------- ------- -------

------- ------- ------- -------

| word  |  eol  |  cut  |  <-   |

|   N   |   M   |   ,   |   .   |

|chgcase|deleol | paste |append |

------- ------- ------- -------

--------------------------------------- ------- -------

bline      |select | enter |

space bar                         |  ->   |  <-   | WATCH OUT FOR

openline    | reset |  sub  | THESE LAST TWO

--------------------------------------- ------- -------

While i was at it i added the h19 function keys f1 to f5 and the help  key

as keys 1 to 5 and 0 with the open/closed apple and button as above.

If you are on a un*x system then it would be better and faster to use  one

of the following termcaps and leave vt52 emulation off.

-------

#From: decvax!cbosgd!cbdkc1!mww Mike Warren

Mm|aepro|apple-aepro|Apple II+ running ASCII Express Pro--vt52:\

:bs:co#80:li#24:up=\EA:nd=\EC:ce=\EK:cl=300^L:cd=\EJ:ho=\EH:\

:cm=\EY%+ %+ :

# The following six apple entries submitted by jtjones@nosc

A0|apple|Apple ][+ with a Videx Ultraterm card in 80x24 mode:\

5

UPDATES FOR APPLE DOS KERMIT

:bs:co#80:li#24:up=^_:nd=^\\:ho=^Y:ce=2^]:cd=20^K:cl=35^L:cm=^^%r%+ %+ :

A1|app96|Apple ][+ with a Videx Ultraterm card in 96x24 mode with extras:\

:bs:co#96:li#24:up=^_:nd=^\\:ho=^Y:ce=2^]:cd=2*^K:cl=40^L:cm=^^%r%+ %+ :\

:am:bw:so=^O:se=^N:

A2|app48|Apple ][+ with a Videx Ultraterm card in 80x48 mode:\

:co#80:li#48:tc=app96:

A8|app128|Apple ][+ with a Videx Ultraterm card in 128x32 mode:\

:co#128#li#32:tc=app96:

A4|aepro2|Apple ][+ with a Videx Ultraterm card running Ascii Express Pro:\

:bs:co#80:li#24:up=\EA:nd=\EC:ho=\EH:ce=2\EK:cd=2*\EJ:cl=2*^L:cm=\EY%+ %+ :

A5|app80|Apple ][+ with a Videx Ultraterm card - same as 'apple' with extras:\

:am:bw:so=^O:se=^N:tc=apple:

#From medin@nosc for Franklin and a Apple super serial card

fr|franklin|franklin 1000/1200 with ace display and apple super serial:\

:am:so=^O:se=^N:do=^J:\

:cl=2^Z0:\

:cm=\036%r%+ %+ :\

:bs:co#80:li#24:up=\037:nd=^U:ho=^Y:ce=2^]:cd=2^K:

#Hayes micro modem(lower case "poke 1784+slot,0" before calling)"medin@nosc"

fm|framm|Franklin 1000/1200 with Hayes micro modem:\

:am:up=20\037:nd=20^U:so=20^O:se=20^N:tc=apple:

-------

Here is a termcap that I received, we have not had a chance to use it yet.

-------------------------------------------------------------------------------

#From stoner@qumix but untried.

ab|apple2e|Apple //e:\

:bl=^G:bw:cd=4*^K:ce=4^]:cl=100^L:co#80:ho=^Y:is=^R^N:kd=^J:kl=^H:\

:kr=^U:ku=^K:li#24:me=^N:mr=^O:ms:nw=100\r:rs=^R^N:se=^N:sf=^W:so=^O:\

:sr=^V:up=^_:\

:bs:nc:

ap|apple2e-p|Apple //e via Pascal:\

:cm=^^%r%+ %+ :tc=apple2e:

-------------------------------------------------------------------------------

0.5.  Problems with loosing characters.

Interupt driven com cards have the best chance of working without loss  of

characters.  However, due to the way apple handles the writing of diskettes and

the many unthinking programers who have written proms that lock  out  interupts

for all kinds of peripheral cards xon&xoff flow control is almost a requirement

for  the  higher baud rates. The //e is especialy vulnerable, I have seen ][+'s

with non apple 80 col displays that work fine at 9600  baud  without  fail  and

without flow control.

0.6.  Prodos errors

Since prodos has so many more error messages than dos 3.3, when you get  a

prodos  error  an attempt will be made to give an explination for the error. In

addition, the actual error returned by prodos will be displayed. You will  have

to  reference a machine language interface manual for the actual meaning of the

error. Following is a partial listing of the  error  and  explinations.  Latter

versions of prodos may change or update the meaning of these errors.

$01     bad system call number

6

UPDATES FOR APPLE DOS KERMIT

$04     bad call parameter count

$25     interupt vector table full

$27     i/o error

$28     no device connected

$2b     disk write protected

$2e     disk switched with file open

$42     eight files already open

$43     invalid reference number

$44     directory not found

$45     volume not found

$46     file not found

$47     duplicate file name

$48     vol full

$49     volume directory full

$4a     incompatable file format,also a prodos directory

$4b     insupported storage type

$4d     position out of range

$4e     file access error or file locked

$50     file already open

$51     directory structure damaged

$52     not a prodos volume

$53     invalid system call parameter

$55     volume control block table full

$56     bad buffer address

$57     duplicate volume

$5a     file structure damaged

0.7.  Com card drivers

Kermit 3 has now been separated into two assemblies. The main routines and

the com card routines. A vector has been set up in low memory for the  two  as-

semblies to communicate. Look at the working com drivers for tips on how to in-

corporate  your  version of the com driver. some things to note: It is probably

best to buffer the input from the remote and to get input characters  from  the

remote  every chance you get. Note the Microtek VS-622 driver, whenever the in-

put is checked for a character and has a character the character  is  put  into

the buffer immeadiatly. Also when the output is checked for ready to output, if

the  card  is  not ready to output then it is checked for a character to input.

All this should help prevent loosing characters.  Following is the current list

of drivers:

p5xxsschex  -  super serial driver(interupt driven)

p6xxmsvhex  -  microtek serial driver (ssc lookalike not interupt driven)

p7xxhmmhex  -  hayes micromodem driver

p8xxacchex  -  apple com card driver - Ike has now initialized within kermit

paxxccshex  -  ccs 7710 com card driver (interupt driven)

0.7.1.  Documentation for the vector area

address - size - provided by - function

1003      byte     main     This is the baud rate index as follows:

3 - 110

4 - 135.4

5 - 150

6 - 300

7 - 600

8 - 1200

7

UPDATES FOR APPLE DOS KERMIT

9 - 1800

10 - 2400

11 - 3600

12 - 4800

13 - 7200

14 - 9600

15 - 19200

for example:

if index is a 6 then line should be 300 baud

1004      byte              unused

1005      word     driver   Address of a null terminated string.

address should point to a capitalized

string of the drivers id

1007      byte     main     Com slot in the form $n0 where n is the slot #.

1008      byte     main     Force initialization flag when 0.

initilization routine should always initialize

when this flag is 0 & then set flag non zero

1009      word     main     Address of the end of kermit main routine.

100b      byte     main     Flow control is on when high bit is set

1020    3 bytes    driver   Jump to initialization routine.

1023    3 bytes    driver   Jump to command routine.  A reg has command

0 - hang up the line

$0b - set baud rate

$0c - set break on the line

$91 - do xon on the line

$93 - do xoff on the line

routine returns false (P reg zero flag)

if unable to do command

1026    3 bytes    driver   Jump to check for input from the line.

routine returns false (P reg zero flag)

if no character on line

1029    3 bytes    driver   Jump to get input character from line.

routine returns character on the line in A reg

102c    3 bytes    driver   Jump to put character in A reg on line.

102f    3 bytes    driver   Jump to reset com driver.

1040    3 bytes    main     Jump to wait routine.A reg contains milliseconds

routine destroys A & X regs

1043    3 bytes    main     Jump to routine to print null terminated string.

X reg contains least significant byte of address

Y reg contains most significant byte of address

routine does not issue a carrage return

1046    3 bytes    main     Jump to routine to read the keyboard.

A reg contains the character read

1049    3 bytes    main     Jump to routine to print carrage rtn & line feed.

104f    3 bytes    main     Jump to routine to set characters parity.

A reg contains the character befor and after

All the routines should return with the "rts" instruction. Routines  which

can  return  a true/false indication should return with the P reg zero flag set

appropiately. That is:a "beq" instruction will branch on a false indication and

the "bne" will branch on a true indication.  Currently the  com  driver  should

start  its  routines  at address $6c00.  If your com driver gets too large then

the bsave address would have to be changed when you are saving  the  binary  to

diskette.

8

UPDATES FOR APPLE DOS KERMIT

0.8.  Loading kermit 3

The main problem exists in getting the hex files onto your disketts  as  a

text  file.  But again that is a test of your creativity. If you have a version

of kermit running then GET or RECIEVE the file as a text file and  you  are  in

business.   We have separated the main routine into two separate files for mail

purposes so EXEC each of the main routines. Use "3d0G" to get  out  of  monitor

back  to  basic for each EXEC.  Your apple (or compatable) will go into monitor

and show you *'s for several minutes. This is the monitor loading the hex  into

binary.  If you get beeps from the monitor its probably because you didnt get a

good copy of the text file.  Now EXEC the com card driver you are going to use.

You will have to get back into basic(aha another test for you,try "3d0G") to do

this. And you will see the monitor loading the com driver. The order of  EXEC's

is  important.   The  com card should be loaded last.  Next get back into basic

and do a "BSAVE kermitname,A$1000,L$5f00" You may have to specify the drive  to

do  this  binary save, with a slot or drive on the end of the BSAVE(aha another

test).  You now run kermit via "BRUN kermitname"

If you want to customize kermit for your equipment. Do  all  your  "set's"

etc  and then do an "exit". Now you should be back in basic. At this point do a

"BSAVE name,A$1000,L$5f00" and when you do a "BRUN name" all your setup will be

remembered.

Since the org is now $1000 if you have been using  kermit  and  then  went

back  to basic for some trivial thing (like catalog) a "CALL 4096" should start

up kermit without having to reload it.

In summary (where nn is the version number):

EXEC p2nnmainhex

EXEC p3nnmainhex

Choose the com card driver you will use. For example p5nnsschex.

EXEC p5nnsschex

BSAVE kermitname,a$1000,l$5f00

And you should be in business. Remember there  is  the  command  HELP  and

whenever  you are into a command a "?" will give you the posible options avail-

able at that point of a command. The escape key will finish typing an option if

it is possible. The syntax of all the commands and options only requires enough

characters to make that command or option unique.

0.9.  Problems

0.9.1.  Installation

NOTE: When using the super serial driver you must have the cards sw6-2 on.

This allows the card to use interupts. The rest of the switches  are  set  from

within kermit. It appears to me that you can run your apple 2 with sw6-2 on and

in  99%  of the cases will cause no problems. This is because dos runs with in-

terupts locked out ("sei" in assembly language) and the program must explicitly

give a "cli" for interupts to work (the super serial driver does).

The microtek driver is a super serial look alike which does not  run  with

interupts.  If  you have trouble with the super serial driver you might try the

9

UPDATES FOR APPLE DOS KERMIT

msv driver.  For you people with the msv-622c card. You  might  try  running  a

jumper  from the UART 6551 pin 26 to the card edge pin 30. This will enable in-

terupts just like the ssc sw2-6.

The prometheus card will work with the apple com driver. However you  will

have  to set the switches on the card for baud etc. Evidently this card can not

be programed by the software. If that is not true then here  is  an  oportunity

for  you to write a better driver. If you do please pass it on for other prome-

theus users.

Unconfirmed reports have it that the apple cat will work  with  the  apple

com driver. Would appreciate a confirmation.

Some have noted the apple com card must be initialized via the "IN#x"  be-

fore starting kermit. Ike has now updated this driver and the initialization is

now done within the apple com driver. Thanks Ike.

0.9.2.  Usage

There is the command HELP and whenever you are into a command a  "?"  will

give  you  the posible options available at that point of a command. The escape

key will finish typing an option if it is possible. The syntax of all the  com-

mands  and  options only requires enough characters to make that command or op-

tion unique.

When using flow control you may appear to hang. Type a ^Q (control Q)  and

that may free you up.

Remember when you use your printer there are a lot of variables here. What

was being sent to the screen now is being sent to your  printer.  If  you  were

emulating  the  vt52  your  printer  may  not know how to handle the escape se-

quences, tabs etc. It may be you can tell the host you are a tty or  some  such

device  that  will give carrage returns etc that your printer can handle.  Some

printers may require the flow control and delay to get readable printing.

File transfer is very dependent upon parity. Make sure the host and  local

parity are the same. Following is one ibm sites method for file transfer:

We have an IBM 3033 and 4381 and use  both  3705/3725  and  7171/Series  1

front  ends.   The  only  difference  in front ends as far as any microcomputer

Kermit is concerned is duplex (local-echo on for the 3705, local-echo  off  for

the 7171).  In Kermit-65 they need to set the following parameters:

BAUD              - whatever is supported.

PARITY            - EVEN for us, maybe MARK for others.

(It is the PARITY which usually causes people problems.)

FLOW              - XON for the 7171, NONE for the 3705.

FLOW DELAY        - 00

0.10.  Caveat

We assume nothing. We insure nothing. User uses at his own risk BUT we are

here to help.

10

UPDATES FOR APPLE DOS KERMIT

Appreciate bug reports, problems and comments.  Thanks to Arnie Groenvel &

Dave Seaman for help in getting the //c screen working correctly.

Ted

address "medin@nosc.arpa"

or   "medin@nosc.mil"

11

###########################################