Apple Emulator Documentation

This is the documentation for the IBM 80x86 Apple ][

emulator. The emulator is not quite complete but since it is a

"for fun" project I thought that I would go ahead and release

what I currently have. The following describes the stage of

completion for each of the Apple ][ emulator sections:

Video         - The emulator will run on a CGA, EGA, or

VGA, however, it looks much better on an

EGA or VGA. All the Apple ][ video modes

are supported except high resoulution mixed

mode with 4 lines of text at the bottom of

the graphics screen. Low resolution and high

resolution graphics are supported.

Keyboard      - The keyboard functions as an Apple ][

keyboard except for the following keys:

F9 - Enter debug mode

F10 - Switch disks A and B

F11,PrtSc - Exit the Apple ][ Emulator

F12,CtrlBreak - Break

ScrlLock  - Change the screen update method

NumLock  - Toggle between keypad and joystick

Pad+ - Increase emulator speed

Pad- - Decrease emulator speed

Some programs run faster depending on the

video update method. Try toggling the scroll

lock to see if the program runs faster.

Joystick      - A PC joystick or numeric keypad can be used

to emulate an apple joystick.  See

JOYSTICK.DOC for more information on joystick

modes, ajustments, etc.

Disk Drive    - Only Apple ][ floppy type drives are supported

now, I need more information on how a Apple

hard disk works. The floppies are emulated by

having disk image files (filenames go in

APPLE.INI).

Langauge Card - A 32k langauge card is supported and can be

accessed via standard langauge card methods.

Serial Card   -

Well there is really a lot more to this but since I'm giving

out the source you can look at it for yourself. I plan on adding a lot

of other features but I'm not sure when I will get around to them.

Until then I hope you enjoy what I have done so far, thanks again.

Randy Spurlock

Apple ][ System Configuration File

The Apple ][ system configuration file is in the following

file format:

[Section 0]             The section can either be CPU or SLOT Number.

Parameter = Value       The parameters are specific to the different

Parameter = Value       device types and are defined below for each

.                  of the device types.

.

.

[Section 1]             The sections can come in any order and slot

.                  numbers can be skipped. If a slot number is

.                  skipped the default value for that slot will

.                  be used, the default values are as follows:

Apple ][ System Configuration Default Values

Slot Number                     Default Device

-----------                     --------------

0                           Language Card (32k)

1                           Super Serial Card

2                           Empty Slot

3                           Empty Slot

4                           Empty Slot

5                           Floppy Disk Controller

6                           Floppy Disk Controller

7                           Cartridge Card (Custom Device)

Apple ][ System CPU Section Parameters

Type = (Normal | Mixed)         If the CPU type is mixed the emulator

can run both 65C02 code and Intel code.

The current CPU execution type can be

switched to Intel by executing the WDM

instruction. Normal type does not allow

execution of Intel code.

Mode = (Normal | Enhanced)      If the CPU mode is enhanced then the

emulator includes the Rockwell 65C02

bit instructions. Normal mode does not

include the bit instructions.

Apple ][ System SLOT Section Parameters

Type = (Language  |             Language card (32k)

Floppy    |             Floppy disk controller

Serial    |             Super serial card

Cartridge |             Cartridge card (Custom device)

Empty)                  Empty slot

Each of the different card types can have there own parameters, listed

below are the different parameters for each of the different card types.

Language Card Parameters

None

Floppy Disk Controller Parameters

A = (Filename)                  File name for disk drive A image

B = (Filename)                  File name for disk drive B image

Super Serial Card Parameters

Port = (COM1 | COM2)            Communications port to use in emulation

of the super serial card.

Baud = (50    |                 Transmit/receive baud rate value.

75    |

110   |

134   |

150   |

300   |

600   |

1200  |

1800  |

2400  |

3600  |

4800  |

7200  |

9600  |

19200)

Stop = (1 | 2)                  Number of stop bits to use, 1.5 for

word lengths of 5.

Length = (5 | 6 | 7 | 8)        Transmit/receive word length in bits.

Parity = (None  |               Transmit/receive parity value.

Odd   |

Even  |

Mark  |

Space)

Pullup = (DCD (+)               Signal values to artificially pull up.

DSR (+)

CTS)

Cartridge Card Parameters

Cartridge = (Filename)          File name of cartridge to load.

Empty Slot Parameters

None