DOWNLOAD(1)         Xinu Programmer's Manual          DOWNLOAD(1)

NAME

download - load program into LSI-11

SYNOPSIS

download [ file ] ...  [ option ] ...

DESCRIPTION

_#D_#o_#w_#n_#l_#o_#a_#d loads an absolute binary program image (PDP-11

a.out format) into the memory of an LSI-11 computer over an

ethernet or an asynchronous line.  After halting the LSI-11

with a break, _#d_#o_#w_#n_#l_#o_#a_#d either places a bootstrap in memory,

and uses it to load the remainder of memory, or jumps to the

resident portion of the downloader.

Loading the object program takes time proportional to the

size of the program itself.  In two seconds, download will

load about 56K bytes over the ethernet, or 1000 bytes over a

serial line.

_#D_#o_#w_#n_#l_#o_#a_#d will normally leave the LSI 11 in ODT.  The -_#a

option requests _#d_#o_#w_#n_#l_#o_#a_#d to auto-start the micro.  The

optional number _#N following -_#a, forces a delay of approxi-

mately _#N seconds before execution, allowing the user to

reconnect the console port before output begins.

The serial-loader bootstrap program is loaded 100 words

beyond the end of the user's object program.  If there is

insufficient memory for the bootstrap and stack, this pro-

gram will fail.  The ethernet downloader stores it's LSI-

resident portion in real page 7, beyond normal object pro-

grams (virtual page 7 is used for memory-mapped I/O).

_#D_#o_#w_#n_#l_#o_#a_#d loads the user's program as specified in the com-

mand line.  If no program name is given, _#a._#o_#u_#t is used.

Options are:

-v   Verbose.  Print details of the communications with the

LSI-11/2.

-s   Silent.  Print no information messages (default is to

print some messages).

-a[_#N]

Autostart.  After loading, start the user's program

running in the LSI-11.  Optionally delay _#N seconds.

-lx  Use LSI number _#x as the LSI 11 for downloading.  If

this option is not specified, download selects the

LSI-11 that the user has on reserve, or a free one if

none is reserved.  E.g., -l9.

Version 6b               Printed 1/12/87                        1

DOWNLOAD(1)         Xinu Programmer's Manual          DOWNLOAD(1)

-d _#f Use file _#f as the downloader program (download will

exec it, after parsing arguments).

-cL  Use LSI in class L (as specified in the _#m_#a_#c_#h_#i_#n_#e_#s data-

base).

-BX  Use Baudrate _#X in communicating with the LSI console.

-h   Hold the LSI, so multiple machines may be allocated to

one user.

-e   Reload the resident portion of the ethernet downloader

(into a page beyond real page 7).

FILES

/dev/LSI.i      LSI 11 device connections, as specified in

machines database

/tmp/xinu/LSI-* Lock files for auto-select

{Xinu-directory}/lib/machinesthe database of available

machines

{Xinu-directory}/lib/sdl.boot.lsiLSI 11 part of serial

bootstrap

{Xinu-directory}/lib/sdl.lsiserial downloader for LSI 11

{Xinu-directory}/lib/edl.lsiethernet downloader for LSI 11

{Xinu-directory}/lib/eload.lsiLSI 11 resident portion of

ethernet downloader

Note: files for other machines have ``lsi'' replaced by the

machine type.

Version 6b               Printed 1/12/87                        2