====================================

DOCUMENTATION FOR MORROW MM300 MODEM

MANUFACTURED BY CTS CORP. (300 BAUD)

====================================

Information supplied by: Ken Cummings

LOCAL (OFFLINE) COMMANDS:

COMMANDS                                 FUNCTION

-------------------------------------------------------------------------

CTL Q <CR>    TURNS THE MODEM ON AND ENTERS THE LOCAL COMMAND

STATE.  BAUD RATE, CHARACTER LENGTH, AND PARITY

ARE AUTOMATICALLY DETECTED.

B <CR>    FROM THE LOCAL COMMAND STATE, THIS COMMAND IS

USED TO RESET THE MODEM INTERFACE CONTROL SOFTWARE

TO THE POWER ON INITIALIZATION POINT.  A CTL Q CR

MUST BE USED FROM THIS POINT TO RE-INITIALIZE THE

MODEM.  THIS COMMAND IS USED TO CHANGE THE MODEM

BAUD RATE.

R <CR>    FROM THE LOCAL COMMAND STATE,  THIS COMMAND RESETS

ALL CONFIGURATION PARAMETERS TO THEIR POWER-ON

DEFAULT CONDITION EXCEPT BITS/CHARACTER.

A <CR>    FROM THE LOCAL COMMAND STATE,  THE MODEM IS PLACED

ON-LINE IN THE ANSWER MODE.  AUTOMATIC DISCONNECT

IS NOT SUPPORTED IN THIS MODE.

O <CR>    FROM THE LOCAL COMMAND STATE, THE MODEM IS PLACED

ON-LINE IN THE ORIGINATE MODE.  AUTOMATIC

DISCONNECT IS NOT SUPPORTED IN THIS MODE.

D <CR>    FROM THE LOCAL COMMAND STATE, THIS COMMAND IS USED

TO INTERROGATE THE PRESENCE OF A DAIL TONE ON THE

TELEPHONE LINE.

TN <CR>   FROM THE LOCAL COMMAND STATE, THE MODEM WILL TONE

DAIL PHONE NUMBER N.

T <CR>    FROM THE LOCAL COMMAND STATE, THE MODEM WILL TONE

DAIL THE PREVIOUSLY STORED NUMBER.

PN <CR>   FROM THE LOCAL COMMAND STATE, THE MODEM WILL PULSE

DAIL THE PHONE NUMBER N.

P <CR>    FROM THE LOCAL COMMAND STATE, THE MODEM WILL PULSE

DAIL THE PREVIOUSLY STORED NUMBER.

ADDITIONAL DAIL PARAMETERS.

A)  THE PHONE CAN BE UP TO 40 CHARACTERS.

B)  THE PHONE NUMBER IS AUTOMATICALLY STORED AS LONG

AS THE MODEM POWER HAS NOT BEEN SWITCHED OFF.

EXECUTING DAIL COMMAND WITHOUT A PHONE NUMBER IN

THE COMMAND STRING WILL INVOKE THE STORED PHONE

NUMBER.

C)  "BLANK", "DASH", AND "SLASH" CHARACTERS BETWEEN

PHONE NUMBER DIGITS ARE IGNORED.

D)  A COMMA CAN BE ENTERED TO CREATE A FOUR SECOND

PAUSE BETWEEN DAILING DIGITS.

E)  A "W" CHARACTER CAN BE IMBEDDED IN THE PHONE

NUMBER TO INSTRUCT THE MODEM DAILER TO WAIT FOR

AN INTERMEDIATE DAIL TONE.

F)  A "T" OR "P" IMBEDDED IN THE DAIL STRING WILL

INSTRUCT THE MODEM TO EITHER TONE OR PULSE DAIL

THE DIGITS IN THE STRING.

G)  ANY CHARACTER ENTRY AFTER CR AND BEFORE THE

REMOTE MODEM HAS ANSWERED WILL HANG UP THE

MODEM AND RETURN TO THE LOCAL COMMAND STATE.

H)  12 DTMF SIGNALS ARE SUPPORTED BY THE MODEM.

THEY ARE 0 TO 9,#,*.

CN=X<CR>      FROM THE LOCAL COMMAND MODE, THIS SEQUENCE CAN BE

USED TO RECONFIGURE VARIOUS CONTROL OPTIONS.

N                     FUNCTION

------------------------------------------------------

1    NUMBER OF RINGBACKS BEFORE THE LINE IS DISCON-

NECTED.  VALUES FOR X RANGE FROM 4 TO F HEX.

2    SEND SPACE DISCONNECT OPTION.  WITH THIS OPTION

ON, A SPACE TRANSMIT SEQUENCE IS INITIATED UPON

DISCONNECT.  THE SPACE SIGNAL TRANSMITS FOR 3.95

SECONDS AND THEN RETURNS TO THE LOCAL COMMAND

STATE.  X=0, THE SPACE OPTION IS OFF.  X=1, THE

SEND SPACE OPTION IS ON.

3    9/10 BITS PER CHARACTER LENGTH OPTION.  THIS

OPTION IS EFFECTIVE IN BOTH SPEED MODES.

9 BPC SELECTION WILL ACCEPT START-STOP CHARACTER

WITH ONE START BIT, ONE STOP BIT, AND SEVEN DATA

BITS.  (IN THE AUTOMATIC INITIALIZATION MODE,

AN ASCII CHARACTER FROM THE INTERFACE WITH NO

PARITY OR MARK PARITY WILL ENABLE 9 BPC.)

10 BPC SELECTION WILL ACCEPT START-STOP CHARACTER

WITH ONE START BIT, ONE STOP BIT, AND EIGHT DATA

BITS.  (IN THE AUTOMATIC INITIALIZATION MODE,

AND ASCII CHARACTER FROM THE INTERFACE WITH EVEN,

ODD, OR SPACE PARITY WILL ENABLE 10 BPC.)

X=0, 9 BPC IS SELECTED. X=1,10 BPC IS SELECTED.

4    AUTO ANSWER RING COUNTER.  IF X=0, THE AUTO

ANSWER MODE IS DISABLED.  IF X=1 TO F HEX, THE

MODEM WILL COUNT THE INCOMMING RING SIGNALS AND

ANSWER THE PHONE AFTER THE SPECIFIED NUMBER OF

RINGS

5    BREAK SEQUENCE TIME INTERAVAL PROGRAMMING.  THE

TIME INTERVAL IS 200 MS TIMES THE VALUE X.  X

HAS A RANGE OF 1 TO F HEX.

L <CR>            FROM THE LOCAL COMMAND STATE, THE MODEM CAN BE

PLACED IN THE ORIGINATE ANALOG LOOPBACK MODE.

ESSENTIALLY, THE MODEM IS IN A NORMAL

COMMUNICATION MODE DURING ANALOG LOOPBACK,

EXCEPT IT IS NOT CONNECTED TO THE TELEPHONE LINE.

ALL ON-LINE COMMAND SEQUENCES ARE SUPPORTED DURING

THE MODE.  THE ON-LINE "E" COMMAND SHOULD BE USED

TO EXIT FROM THIS MODE.

ON-LINE COMMANDS:

THE ON-LINE COMMAND STATE PROVIDES A MECHANISM WHERE THE CONTROL

PARAMETERS OF THE MODEM CAN BE CHANGED WHILE THE PHONE LINE IS STILL

CONNECTED TO THE REMOTE MODEM.  ONLY A LIMITED NUMBER ARE SUPPORTED

IN THIS MODE.  WHILE THE MODEM IS IN THE ON-LINE COMMAND STATE, DATA

RECIEVED FROM THE REMOTE MODEM IS LOST.  COMMANDS SENT TO THE MODEM

WHILE IN THIS STATE ARE TRANSPARENT TO THE REMOTE MODEM.

TO ENTER THE ON-LINE COMMAND STATE THE INTERFACE MUST ISSUE THE

FOLLOWING SEQUENCE:

CTL T@@@

AFTER ISSUING THE CTL T@@@ SEQUENCE, THE MODEM WILL SEND THE PROMPT

"8".  A 50 MS MINIMUM DELAY MUST BE INCORPORATED BETWEEN THE LAST @

CHARACTER AND THE COMMAND CHARACTER.  AFTER EXECUTING ANY ON-LINE

COMMAND, THE MODEM WILL RETURN TO THE COMMUNICATION MODE.  IF THE

MODEM DETECTS A COMMAND ERROR SEQUENCE, THE MODEM WILL ISSUE THE

PROMPT "E" AND RETURN TO THE COMMUNICATION MODE.   AS AN ADDITIONAL

NOTE, THE CTL T@@@ SEQUENCE IS PASSED THROUGH THE MODEM TO THE REMOTE

STATION.  HOWEVER, THE COMMAND CHARACTER AND CARRIAGE RETURN SEQUENCE

IS TRANSPARENT TO THE REMOTE STATION.

THE FOLLOWING IS A LIST OF ON-LINE CONTROL COMMANDS.

COMMAND                           FUNCTION

-------------------------------------------------------------------------

B <CR>    FROM THE ON-LINE COMMAND STATE, THE MODEM WILL SEND A SPACE

INTERVAL WHICH IS 200MS TIMES THE VALUE STORED IN THE

CONFIGURATION COMMAND (C6).

O <CR>    FROM THE ON-LINE COMMAND STATE, THE CURRENT PROTOCOL

MODE IS SET TO ORIGINATE.

A <CR>    FROM THE ON-LINE COMMAND STATE, THE CURRENT PROTOCOL

MODE IS SET TO ANSWER.

H <CR>    FROM THE ON-LINE COMMAND STATE, THE MODEM WILL DISCONNECT

FROM THE PHONE LINE AND RETURN TO LOCAL COMMAND STATE.

RETURN STATUS CODES:

CODE SEQUENCE                      FUNCTION

-------------------------------------------------------------------------

0           MODEM READY - LOCAL COMMAND STATE

1           DISCONNECT FROM PHONE LINE

2           ON-LINE ORIGINATE

3           NO ANSWER FROM REMOTE MODEM

4           BUSY SIGNAL DETECTED

5           NO CARRIER TONES

6           DIAL TONE DETECTED

7           NO DIAL TONE DETECTED

8           MODEM READY - ON-LINE COMMAND STATE

E           COMMAND ERROR

C           RETURN TO COMMUNICATIONS MODE FROM ON-LINE COMMAND

L           LOW SPEED (300 BAUD)

POWER ON-PARAMETER DEFAULTS

N CODE       X VALUE              FUNCTION

-------------------------------------------------------------------------

1             8            NUMBER OF RING BACKS

2             0            SEND SPACE DISCONNECT OPTION IS OFF

3           AUTO           9/10 BITS/CHARACTER

4             0            AUTO ANSWER FUNCTION IS OFF

5             5            BREAK SEQUENCE TIME INTERVAL